Flood

Jupiter Hazard Module — Beacon System Reference

Status: In Progress

# Overview

Flood is a high-priority Jupiter module with multiple distinct subtypes requiring different modeling approaches. The architectural principle is a swappable trigger module feeding a shared terrain vulnerability engine.

# Flood Type Taxonomy

| Type | Primary Driver | Time Scale | Key Predictors | Spatial Pattern |
| --- | --- | --- | --- | --- |
| Riverine flash | Intense rainfall in watershed | Minutes–hours | Upstream precip, channel geometry, HAND | Linear (follows channels) |
| Urban pluvial | Local rainfall > drainage capacity | Minutes–hours | Impervious %, drain capacity, local depressions | Patchy (low spots, intersections) |
| River flooding | Sustained rainfall / snowmelt | Days–weeks | Upstream gauge levels, soil saturation | Linear (floodplain inundation) |
| Hurricane/coastal | Storm surge + rain + waves | Hours | Storm track, pressure, bathymetry | Coastal gradient + inland rain |
| Dam/levee failure | Structural failure | Minutes–hours | Reservoir level, breach geometry | Downstream channel |
| Tsunami inundation | Seismic event | Minutes | Earthquake mag/location, bathymetry | Coastal runup patterns |

Terrain vulnerability component is largely shared across types. Trigger modules are swappable.

# Core Modeling Approach

## HAND (Height Above Nearest Drainage)

Primary terrain vulnerability index. For each grid cell, computes vertical distance to the nearest river channel following flow paths.

- Computed from DEM (bare earth) — USGS 3DEP LiDAR for US at 1m resolution

- HAND lookup: "gauge at 15ft → flood all cells with HAND < X meters"

- Rating curves (stage → discharge) from USGS or HEC-RAS hydraulic modeling

- Pre-computed FEMA inundation libraries available for many gauged rivers

## Modeling Spectrum

| Approach | Accuracy | Speed | Use Case |
| --- | --- | --- | --- |
| Static bathtub (HAND) | Low-medium | Instant | Storm surge, slow-rise flooding, initial alert |
| 1D hydraulic (HEC-RAS) | Medium | Fast | Riverine flooding, channel flow |
| 2D hydrodynamic | High | Slow (parallelizable) | Flash flood timing, velocity, depth evolution |
| Pre-computed scenario library | High (for covered scenarios) | Instant | Known events at gauged rivers |

Beacon target: hybrid — pre-computed inundation libraries for known scenarios + simplified real-time modeling for active events. Critical output is arrival time and depth at specific user location, not just extent.

## Flash Flood — Specific Challenges

Hardest case: ungauged small catchments responding to intense rainfall faster than traditional warning systems. Hurricane Helene type scenario.

- Gauges often absent on small streams where most flash floods occur

- Flood arrives before gauge data propagates

- Gauges can be destroyed by the flood itself

- MRMS (Multi-Radar Multi-Sensor) for real-time radar-based precipitation input

- Rainfall-runoff modeling: rational method, SCS curve number, or HEC-HMS for larger watersheds

## Surrogate ML Model (Long-Term)

Train surrogates on HEC-RAS or GeoClaw runs across varying precipitation intensity, watershed saturation, and upstream conditions. User reports ("water rising at this intersection," "road flooded") feed directly into data assimilation — direct observations of the prediction target.

# Data Sources

| Source | What It Provides | Update Frequency |
| --- | --- | --- |
| USGS Water Services API | Real-time stream gauge levels and discharge (~10,000 US stations) | 15-minute polling |
| NOAA MRMS | Real-time radar precipitation estimates | Sub-hourly |
| NOAA AHPS | Flood forecasts at gauged rivers | Operational |
| GloFAS | Global river discharge forecasts (modeled, not observed) | Daily |
| USACE reservoirs | Army Corps reservoir levels and releases | Hourly |
| FEMA flood zones | 100-year/500-year static zone polygons | Static |
| HydroSHEDS | Global hydrography, channel network, watershed boundaries | Static |
| USGS 3DEP LiDAR | Bare-earth DEM for HAND computation, building detection | Static (high res) |
| NWS river forecasts | Stage forecasts at gauged locations | Operational |

# Atlas Layer Integration

- HAND layer (pre-computed from DEM)

- FEMA flood zones (static reference)

- Stream gauge locations + drainage area metadata

- Watershed boundaries

- Building footprints with first-floor elevation (from LiDAR nDSM)

- Road elevation profiles (passability during flooding)

- Dam and levee locations with breach inundation zones

- Reservoir levels (real-time, from USACE)

# Alert Logic (Argus)

## What Beacon Delivers

- Not just "you're in a flood zone" — "you are downriver of a flash flood currently forming at X watershed"

- Estimated arrival time at user location

- Personalized routing: avoid roads X, Y, Z — take route A instead

- Electrocution risk overlay: floodwater + downed power lines (Hurricane Katrina scenario)

- "Water rising or receding?" — contextual update cadence based on gauge trends

## BSP Encoding (Hermes)

- C20: Flash flood — occurring

- C21/C22: Flash flood warning/watch

- C2C/C2D: Dam failure imminent/occurred

- C2E: Levee breach

- C2G/C2H: Water rising/receding

# Routing Constraints (Exodus)

- Water depth threshold by vehicle type (sedan vs. SUV vs. truck)

- Flow velocity: fast-moving shallow water is more dangerous than deep still water

- Road surface type affects passability threshold

- Real-time crowdsourced road flood reports from Agora

- Conservative fallback: unknown vehicle → lowest passability threshold