Earthquake

Jupiter Hazard Module — Beacon System Reference

Status: Planned

# Overview

Earthquake is uniquely constrained as a hazard: seismic shaking arrives faster than any warning system can respond at the source. Beacon's value is a combination of pre-event risk awareness, the ShakeAlert integration for seconds of lead time for users away from the epicenter, and post-event guidance.

# The Warning Timing Problem

Seismic waves travel at 3–7 km/s. By the time USGS publishes location and magnitude, shaking is already over in the epicentral region. This is fundamentally different from flood or tsunami.

## What IS Possible

- ShakeAlert (USGS): detects P-wave at seismometers near epicenter → alerts users farther away before S-wave (strong shaking) arrives. Gap = 10–60 seconds depending on distance.

- Android Earthquake Alerts System (Google): crowdsourced P-wave detection across Android accelerometers. Operational since 2020. Could inform Beacon.

- Post-shaking guidance: within minutes, USGS publishes ShakeMaps showing frequency content and intensity distribution → building-height-specific guidance

## What Is NOT Possible (Currently)

- Real-time "get out now" with more than seconds of warning at source

- On-the-fly frequency content modeling from public APIs (raw seismometer data not public)

- Warning before shaking at the epicenter

# ShakeAlert Integration

ShakeAlert publishes processed alerts (not raw seismometer data). Beacon integrates:

- Alert receipt → push notification to users in expected shaking zone

- Message: "Strong earthquake approaching — take cover now"

- Building-height-specific guidance: 1-2 Hz resonance affects mid-rise (6-15 stories); longer period affects high-rise

- Post-shaking: "Earthquake just hit — wait for aftershocks before exiting, check for structural damage before re-entering"

# Pre-Event Risk Layer

- Seismic hazard maps (USGS NSHM): 2%, 10% probability of exceedance in 50 years

- Fault line locations and characterization (USGS Quaternary Fault Database)

- Liquefaction susceptibility zones (soil type + water table)

- Landslide susceptibility triggered by seismic shaking

- Building vulnerability by construction type and year (critical for post-event damage modeling)

# Post-Event Guidance

## ShakeMap Integration

- USGS ShakeMaps available within minutes of event

- Intensity (MMI), PGA, PGV, spectral acceleration at user location

- Beacon translates: "Your area experienced strong shaking (MMI VII). Wood-frame houses in this range typically have X damage pattern."

- Aftershock forecast: USGS operational aftershock forecasting → "60% chance of M5+ in next week"

## Tsunami Trigger

- Large coastal/subduction earthquakes hand off immediately to Tsunami module

- M7.5+ at depth <70km near subduction zone → tsunami alert pipeline activates

- See Tsunami document for full logic

## Infrastructure Assessment

- Bridge and overpass vulnerability lookup (Caltrans, FHWA bridge inspection data)

- Route modification in Exodus: flag potentially damaged bridges post-event

- Utility alert: gas line ruptures (crowdsourced + utility operator feed)

# Data Sources

| Source | What It Provides |
| --- | --- |
| USGS Earthquake API | Real-time event detection: magnitude, depth, location |
| ShakeAlert | Processed early warning alerts |
| USGS ShakeMap | Post-event shaking intensity distribution |
| USGS NSHM | Probabilistic seismic hazard maps |
| USGS Quaternary Fault Database | Fault line locations and slip rates |
| USGS Slab2.0 | Subduction zone geometry (for tsunami trigger) |
| OpenQuake / HAZUS | Building fragility and loss modeling |
| Android P-wave detection | Crowdsourced early warning signal (informational) |

# BSP Encoding (Hermes)

- D00–D06: Earthquake felt / minor / light / moderate / strong / major / great

- D07: Aftershock

- D08: Earthquake swarm

- D09/D0A: Structure/infrastructure damage

- D0C: Liquefaction reported