Sensor-Based Systems Architecture v4

Reorganized: unified positioning module, risk flags vs event flags, tiers + event protocols, community features

Companion files: SENSOR_SYSTEMS_MASTER.xlsx (full script index, inputs/outputs, efficacy, grouping), SENSOR_FLAG_TABLES.xlsx (flag definitions, event tables, mesh protocol)

# 1. Design Principles

Positioning is centralized. One engine fuses all sources (GPS, WiFi, BLE mesh, IMU dead reckoning). All modules that need position call this engine. Positioning also handles map layer checks, geofencing, passive phone detection, and all tracking use cases.

Risk flags adjust awareness. They change polling tiers but never alert users. If you are on a mountain, avalanche sensors are on. If rain falls upriver, flood polling escalates. No warning needed.

Event flags mean something happened. They trigger validation, model feedback, mesh broadcast, notifications, and hazard lifecycle management.

Polling uses 4 tiers (T0-T3) for baseline rates. Event protocols override specific sensors during specific events, replacing the old T4 emergency tier. A trapped person does not need 10Hz GPS. A person about to be buried needs a 15-second GPS burst, not sustained max polling.

The mesh positioning system is unified across all scenarios: indoor evacuation, outdoor evacuation, post-collapse SAR, and normal operation. Beacon users triangulate off each other using IMU dead reckoning corrected by BLE ranging. GPS and WiFi provide absolute anchors when available. Non-Beacon phones are detected passively.

# 2. Module Map

Data flows UP through layers. Polling commands flow DOWN. Risk flags flow INTO the tier controller. Event protocols OVERRIDE specific sensor rates during events.

POSITIONING: core engine, map layers, tracking, passive detection, geofencing (31 files)

Layer 0: Device Abstraction - capability profiles, platform adapters (4 files)

Layer 1: Sensor Streams - normalized data (14 files)

Layer 2: Polling Controller - tier state machine + event protocol runner (5 files)

Layer 2.5: Risk Engine - geospatial risk + contextual + warnings + aggregator (15 files)

Layer 3: Classifiers - activity, transit, recreation, environment, connectivity (17 files)

Layer 4: Event Engine - 39 triggers + 6 event protocols (42 files)

Layer 5: Validation + Mesh - secondary checks, peers, model cross-ref, BLE mesh (12 files)

Layer 6: Hazard Lifecycle - records, active index, escalation, resolution (5 files)

Layer 7: Notifications + Map + Community - channels, map UI, help/shelter/reporting (22 files)

Layer 8: Privacy + Compliance (9 files)

External: Plowing + Integrations (~14 files)

# 3. Positioning Module

folder: positioning/

One system, one principle: devices triangulate off each other, always. What changes is which extra reference signals are available.

## Core Engine (positioning/core/)

| Module | Purpose |
| --- | --- |
| position_engine.js | Unified fusion: GPS, WiFi RTT, BLE ranging, IMU dead reckoning, cell tower. Outputs best position with confidence. |
| imu_dead_reckoning.js | Step + heading from accel/gyro/mag. Cm-level accuracy over short distances before drift. |
| ble_ranging.js | BLE RSSI distance estimation between Beacon devices. Corrects IMU drift. |
| wifi_rtt_ranging.js | WiFi 802.11mc RTT positioning. Sub-meter when available. |
| gnss_processor.js | Multi-constellation GNSS. Dual-frequency L1+L5 on newer phones. |
| altitude_engine.js | Fused altitude: baro (primary indoor) + GNSS (calibration) + floor number. |
| mesh_positioning.js | Device-to-device: join mesh, share positions, correct drift. Same system for evac, normal, post-collapse. |
| position_confidence.js | Score each source, weight for fusion. Adapts to conditions. |

Building with power: mesh + WiFi APs + BLE beacons = best accuracy. Building without power: mesh only. Outdoors: mesh + GPS. Post-collapse: mesh continues, SAR devices join with GPS anchors.

## Map Layers (positioning/map_layers/)

| Module | Purpose |
| --- | --- |
| layer_checker.js | Check position against ALL map layers. Output risk context to risk engine. |
| network_snap.js | Snap GPS to road or rail network. Merged from road_snap + rail_snap. |
| water_body_layer.js | River, lake, ocean proximity. |
| building_footprint_layer.js | Building polygon + ID match. |
| terrain_layer.js | DEM: elevation, slope, aspect, treeline. |
| ski_run_layer.js | Ski runs, lifts, difficulty. |
| infrastructure_layer.js | Bridges, tunnels, dams, chemical facilities, power plants, explosive storage, nuclear. |
| hazard_zone_layer.js | Faults, flood plains, fire perimeters, volcanic zones, blast radii, radiation, lahar. |
| tile_manager.js | Cache, fetch, expire, predict-ahead. |

## Tracking (positioning/tracking/)

| Module | Purpose |
| --- | --- |
| track_logger.js | Continuous position logging. Base for all tracking. |
| evacuation_tracker.js | Indoor + outdoor evacuation. Mesh when GPS unavailable. |
| sar_coordination.js | SAR map: mesh positions + passive phone counts + zones. |
| favor_tracker.js | Track user doing favor. Check-in. Alert if off-route. |
| ski_run_recorder.js | Record ski run. Prompt condition report at bottom. |
| drift_tracker.js | Water/debris drift for rescue prediction. |

## Passive Detection (positioning/passive_detection/)

Every phone broadcasts BLE. Beacon users detect non-Beacon phones passively. Multiple Beacon devices triangulate every BLE source. SAR gets count + approximate positions for all phones in a debris field.

| Module | Purpose |
| --- | --- |
| passive_phone_scanner.js | Scan for non-Beacon BLE/WiFi. Filter by device class. |
| phone_counter.js | Deduplicate MACs + count unique phones. |
| passive_triangulator.js | Triangulate passive phones from multiple mesh nodes. |

## Geofencing (positioning/geofencing/)

| Module | Purpose |
| --- | --- |
| geofence_engine.js | Core: create, check, enter/exit for arbitrary polygons. |
| police_area_query.js | LE geofences area. Users prompted to report observations. |
| utility_area_query.js | Utility geofences area. Users asked: power? water? gas? |
| emergency_area_broadcast.js | Auto-broadcast emergency to radius. Goes to nearby + EMS. |
| shelter_radius_broadcast.js | "I have room in shelter/car." Radius-based to opted-in users. |

# 4. Polling: Tiers + Event Protocols

4 base tiers set ambient rates. Event protocols override specific sensors during events. No T4. Intelligent per-event overrides.

## Base Tiers

| Tier | Accel | Gyro | GNSS | Baro | BLE | Drain/hr |
| --- | --- | --- | --- | --- | --- | --- |
| T0: SLEEP | Coproc wake | Off | Off | Off | Off | 0.3-0.5% |
| T1: IDLE | 10Hz coproc | Off | 0.05Hz | 0.1Hz | 60s scan | 0.8-1.2% |
| T2: AWARE | 50Hz | Off* | 1Hz | 1Hz | 10s scan | 2-3.5% |
| T3: ACTIVE | 100Hz | 50Hz | 5Hz | 5Hz | 5s scan | 6-10% |

*Gyro on at T2 only for cycling/skiing.

## Event Protocols (replace T4)

| Protocol | When | What | Duration |
| --- | --- | --- | --- |
| GPS Burst | Life-threat/environmental trigger | GNSS 10Hz 5-15s. Cache pre-event coord. | 5-15s |
| Indoor Cache | Earthquake/collapse while indoor | Snapshot mesh position, floor, building_id. | 2-3s |
| Trapped: Conserve | TRAPPED + no SAR nearby | BLE listen 5s, advertise 2s, accel 10Hz, GPS off. 2-3%/hr. | Hours |
| Trapped: Found | SAR beacon detected | BLE max, audio beep, GPS retry, accel 100Hz. 15-20%/hr. | Minutes |
| SAR Coordination | User declares SAR | BLE continuous, GPS 10Hz, passive scanner. 12-20%/hr. | SAR duration |
| Drift Tracking | Water/avalanche + moving | GPS 10Hz, baro 10Hz, accel 100Hz. 12-20%/hr. | Until stationary |
| Proximity Beep | SAR Beacon user near trapped | Audio modulated by RSSI. Closer = faster. | While in range |

# 5. Classifiers

17 classifiers, down from 32 detectors. Merged overlapping functions.

| Classifier | Replaces | Outputs |
| --- | --- | --- |
| vehicle_classifier | driving + speed + parking | PARKED/DRIVING/STOPPED, speed |
| locomotion_classifier | walking + running | WALKING/RUNNING, cadence |
| cycling_classifier | cycling (simplified) | CYCLING, speed |
| public_transit_classifier | train + bus + subway | TRAIN/BUS/SUBWAY, route_id |
| flight_classifier | plane | IN_FLIGHT, phase |
| elevator_classifier | elevator | IN_ELEVATOR, direction, floors |
| ski_recreation_classifier | ski/snowboard + chairlift | SKIING/BOARDING/CHAIRLIFT |
| water_activity_classifier | water_activity | ON_WATER, type |
| elevation_gain_classifier | climbing (renamed) | GAINING_ELEVATION, rate |
| inactivity_classifier | stationary + sleeping + dwell | STATIONARY/DWELLING/SLEEPING |
| building_classifier | indoor/outdoor + building entry | INDOOR/OUTDOOR, building_id |
| enclosed_space_classifier | underground + tunnel + parking | UNDERGROUND/TUNNEL/PARKING |
| crowd_density_classifier | crowd_density (kept) | density, peer_count |
| power_grid_inferrer | kept | outage_detected, area |
| connectivity_state_machine | kept | WIFI/CELL/SAT/OFFLINE |
| baro_baseline_tracker | kept | baseline, deviation |
| weather_data_client | wind_estimator renamed + AQ cut | wind, AQ, temp, precip |

Cut: traffic_density_estimator (unreliable single-device), air_quality_inferrer (cant measure AQ from phone sensors).

# 6. Risk Flags

Adjust polling tiers. Never alert. Full definitions in SENSOR_SYSTEMS_MASTER.xlsx > Risk Flags sheet.

| Category | Count | Examples |
| --- | --- | --- |
| Device State | 11 | BATTERY_CRITICAL, OFFLINE, SATELLITE_ONLY, GPS_DEGRADED |
| Activity | 16 | DRIVING, WALKING, SKIING, ON_WATER, ON_TRAIN, SLEEPING |
| Geospatial (always on) | 22 | ON_AVALANCHE_TERRAIN, ON_FAULT_LINE, NEAR_EXPLOSIVE_STORAGE, NEAR_RADIATION_SOURCE |
| Hazard Warnings | 8 | TORNADO_WARNING, TSUNAMI_WARNING, AVALANCHE_WARNING |
| User-Declared | 6 | EVACUATING, TRAPPED, SAR, BACKCOUNTRY |
| Contextual | 5 | MASS_GATHERING, UNDERGROUND, ON_BRIDGE |

New: NEAR_EXPLOSIVE_STORAGE, NEAR_RADIATION_SOURCE, NEAR_CHEMICAL_STORAGE, NEAR_LAVA_ZONE.

# 7. Event Engine

39 triggers. New: explosion (#18), radiation event (#19), lava flow (#20), chemical spill (#21).

| Category | Count | Triggers |
| --- | --- | --- |
| Life Threat | 12 | Crash, rollover, hard brake, ped fall, high fall, shock, fire, water, cold, heat, crush, asphyx |
| Environmental | 9 | Earthquake, landslide, tsunami, infra fail, hazmat, EXPLOSION, RADIATION, LAVA, CHEMICAL SPILL |
| Infrastructure | 4 | Building collapse, gas leak, power outage, road/bridge damage |
| Device | 3 | SOS, last gasp, signal lost |
| Tracking | 4 | Route deviation, stationary, geofence exit, speed anomaly |
| Rescue | 3 | Beacon detected, no response, cluster emergency |
| Compound + State | 4+ | Multi-hazard logic + state transitions |

Peer validation: building collapse requires multiple users showing floor anomalies. Single user = personal fall. Earthquakes need 3+ correlated devices.

# 8. Community Module

| Module | Purpose |
| --- | --- |
| help_request.js | Ask nearby people for help. Non-responders hidden from requester. |
| shelter_offer.js | "I have room" broadcast to radius. |
| condition_reporter.js | Report ski/road/power conditions. Context-prompted. |
| authority_query_responder.js | Respond to police/utility geofence queries. |
| loved_ones_status.js | Broadcast risk/status to groups/contacts/EMS. |
| map_tag_submitter.js | Submit condition tags to live map. |

# 9. Plowing Module

| Module | Purpose |
| --- | --- |
| road_snow_estimator.js | Estimate snow from driving data (speed reduction, traction loss). |
| plow_route_tracker.js | Track plow path + coverage. |
| road_condition_aggregator.js | Aggregate reports + sensor data into road condition map. |

# 10. Mesh + SAR

BLE mesh is one system. Devices triangulate off each other using IMU corrected by BLE ranging. Building with power adds WiFi/BLE beacons. Post-collapse: mesh continues, SAR devices join with GPS anchors.

SAR map: buried Beacon devices know positions relative to each other. SAR devices add absolute reference. Non-Beacon phones are passive BLE dots. SAR gets: total count + zones + detailed context for Beacon users.

Trapped protocol: Conserve (BLE listen, 2-3%/hr, survive 20+ hours) until SAR detected, then Found (max power, audio beep, 15-20%/hr). Proximity beep: RSSI-modulated audio guides SAR team.

# 11. File Count

| Area | Files |
| --- | --- |
| Positioning | 31 |
| Sensors | 27 |
| Classifiers | 17 |
| Risk Flags | 15 |
| Event Engine | 65 |
| Community | 6 |
| Plowing | 3 |
| Notifications | 13 |
| Privacy | 9 |
| TOTAL | ~190 |

Down from ~245 in v3. Merged 32 detectors into 17 classifiers. Cut traffic_density + air_quality. Consolidated duplicates.