Trigger #3

Pedestrian fall

Category: life_threat

sensor_driven_triggers/life_threat/pedestrian_fall/

# 1. Trigger Overview

## 1.1 What Triggers This Hazard

Accelerometer detects freefall (near-zero acceleration >200ms) followed by impact spike >2g followed by no significant movement for 60 seconds. GPS speed must be <5mph (pedestrian pace).

## 1.2 Associated Events Required

None required. Standalone. If active weather warning (ice storm, high wind) in area, severity auto-escalated.

## 1.3 Who Gets Notified

| Party | Notified | Reason |
| --- | --- | --- |
| Police | No | N/A |
| Fire | No | N/A |
| Ems | Yes | Non-responsive person, possible head injury or cardiac event |
| Utilities | No | N/A |
| Transit | No | N/A |
| Emergency Managers | No | Only if repeated falls at same location |
| Nearby Users | Yes | First-aid capable users within 200m |
| Nearby Drivers | No | N/A |

## 1.4 Polling Protocol During Hazard

After freefall+impact detected, accelerometer continues at 100Hz monitoring for movement. GPS polls at 1Hz for location lock. 60s no-movement window is the confirmation period.

| Sensor | Frequency | Duration |
| --- | --- | --- |
| Accelerometer | 100Hz | 60s movement watch |
| GPS | 1Hz | Until resolved |

## 1.5 End of Hazard Conditions

Hazard ends when: (1) user responds to confirmation prompt, (2) significant movement detected after initial stillness, (3) EMS marks resolved, or (4) emergency contact confirms user is OK.

## 1.6 Mesh Status

Mesh stays in current state. If user non-responsive for 60s, mesh broadcasts BLE distress beacon at elevated power for 5 minutes to alert nearby mesh devices.

## 1.7 Platform Support

| Platform | Supported | Notes |
| --- | --- | --- |
| Android | Yes | Accelerometer 100Hz, motion activity API 1Hz |
| iOS | Yes | CoreMotion 100Hz, activity recognizer 1Hz |

# 2. Script Architecture

## 2.1 Sensor Watcher: watch_pedestrian_fall.js

File: sensor_driven_triggers/life_threat/pedestrian_fall/watch_pedestrian_fall.js

Reads accelerometer at 100Hz. Detects freefall phase: magnitude <0.3g for >200ms. Detects impact phase: magnitude >2g within 500ms of freefall end. Starts 60s no-movement monitor (variance <0.1g). GPS pre-filter: speed <5mph. Emits FALL_DETECTED on freefall+impact, FALL_CONFIRMED after 60s no movement.

Inputs

- accelerometer_stream

- gps_location_stream

- step_counter_stream

Outputs

- FALL_DETECTED event

- FALL_CONFIRMED event (after 60s stillness)

Related Code

- sensor_polling/accelerometer_stream.js

- sensor_polling/gps_location_stream.js

- sensor_polling/step_counter_stream.js

- sensor_driven_triggers/life_threat/pedestrian_fall/detect_freefall_phase.js

- sensor_driven_triggers/life_threat/pedestrian_fall/detect_impact_phase.js

- sensor_driven_triggers/life_threat/pedestrian_fall/monitor_post_impact_movement.js

- sensor_driven_triggers/life_threat/pedestrian_fall/estimate_fall_height.js

## 2.2 Validation & Sync: validate_pedestrian_fall.js

File: sensor_driven_triggers/life_threat/pedestrian_fall/validate_pedestrian_fall.js

On FALL_DETECTED, shows audible 30s confirmation countdown. If user responds, logs false alarm. If no response AND 60s no-movement confirmed, validates: not in vehicle, not in bed (time-of-day + location heuristic). No mesh corroboration needed (individual event). Emits FALL_VALIDATED.

Inputs

- FALL_DETECTED event

- FALL_CONFIRMED event

- user_activity_context

Outputs

- FALL_VALIDATED event or FALL_DISMISSED event

Related Code

- sensor_driven_triggers/shared/user_confirmation_countdown.js

- sensor_driven_triggers/shared/check_known_activity.js

- sensor_driven_triggers/life_threat/pedestrian_fall/group_pedestrian_fall_alerts.js

## 2.3 Status Log: log_pedestrian_fall_hazard.js

File: sensor_driven_triggers/life_threat/pedestrian_fall/log_pedestrian_fall_hazard.js

On FALL_VALIDATED, creates hazard record with freefall duration, impact force, estimated fall height, no-movement duration, location. Generates hazard_id, inserts into active_hazards. If repeated falls at same location (within 50m, past 30 days), flags as hazardous_location for infrastructure review.

Inputs

- FALL_VALIDATED event

Outputs

- hazard_id

- hazard_status_log record

- optional hazardous_location flag

Related Code

- data/hazard_status_log.js

- data/active_hazards.js

- sensor_driven_triggers/shared/flag_hazardous_location.js

## 2.4 Map Icons & Notifications: notify_pedestrian_fall.js

File: sensor_driven_triggers/life_threat/pedestrian_fall/notify_pedestrian_fall.js

On FALL_VALIDATED, places fall icon on map. EMS standard: 'Pedestrian fall detected. Non-responsive 60s+. Location: [address]. Estimated fall height: [X]m.' Nearby users (200m): 'Person may need help [distance] away. Tap for directions.' Emergency contacts: 'Your emergency contact [name] may have fallen and is not responding. Location: [address].' No driver or police notification unless in roadway.

Inputs

- FALL_VALIDATED event

- hazard_id

- emergency_contacts_list

Outputs

- map_icon_event

- ems_notification

- nearby_user_notification

- emergency_contact_notification

Related Code

- notifications/channels/notify_ems.js

- notifications/channels/notify_nearby_users.js

- notifications/channels/notify_emergency_contacts.js

- ui/maps/place_hazard_icon.js

- ui/maps/fall_event_pin.jsx

- geospatial/reverse_geocode.js

- notifications/templates/fall_ems_template.js

- notifications/templates/fall_nearby_template.js

- notifications/templates/fall_contact_template.js

# 3. Alert Grouping

Function: filterFallFalsePositives

File: sensor_driven_triggers/life_threat/pedestrian_fall/fall_filter.js

- Verify activity label confirms "fall" not just accel variance

- Check GPS elevation change to distinguish falls from bouncing

- Suppress alerts if user moves within 5s of impact

# 4. Data Tables

## fall_events

Purpose: Log detected pedestrian falls

Fields

- user_id

- timestamp_fall

- impact_g

- freefall_duration_ms

- stillness_duration_s

- latitude

- longitude

- surface_type

## fall_sensor_traces

Purpose: Raw accelerometer data during fall

Fields

- fall_event_id

- accel_magnitude_array

- activity_labels_array

- timestamps_ms

## hazard_status_log

Purpose: Central hazard record for this trigger event

Fields

- hazard_id UUID PK

- trigger_id INT

- trigger_type TEXT

- severity ENUM(low,medium,high,critical)

- status ENUM(detected,confirmed,active,resolved,archived)

- location GEOMETRY(Point,4326)

- device_anonymous_id TEXT

- timestamp TIMESTAMPTZ

- parent_emergency_id UUID FK nullable

- sensor_snapshot_ref TEXT

- metadata_json JSONB

- created_at TIMESTAMPTZ

- resolved_at TIMESTAMPTZ nullable

## active_hazards

Purpose: Index of all currently active hazards across all trigger types

Fields

- hazard_id UUID PK FK→hazard_status_log

- trigger_type TEXT

- priority INT

- region GEOMETRY(Polygon,4326)

- device_count INT

- last_updated TIMESTAMPTZ

# 5. Hazard Model Integration

| Model | Integration | Storage |
| --- | --- | --- |
| Terrain elevation database | Validate fall from elevation change (not just acceleration) | Cache nearby elevation 10min |

# 6. Data Storage Practices

## Individual Event

fall_events table with user_id partition

## Emergency Linkage

Streamed to regional EMS in real-time

## Retention

Raw traces 24h if no alert sent, 7 days if alert sent; event records 90 days

## Archival

Anonymized fall statistics for 1 year for epidemiological research

# 7. Required Permissions

- sensor:accelerometer:read

- sensor:activity:read

- sensor:gps:read

- notification:send:ems

- notification:send:public

- notification:send:admin

- database:fall_events:write

- database:fall_sensor_traces:write

- hazard_models:terrain:query

# 8. Settings Toggle

Pedestrian Fall Detection: Monitors for rapid acceleration drops and impacts followed by prolonged immobility to detect potential falls.

# 9. Legal Concerns

- Continuous accelerometer sampling for fall detection

- False positive alerts summoning EMS to healthy individuals

- Liability if fall not detected due to sensor limitations or device placement

- Activity recognition privacy (Apple/Google Activity API privacy terms)

- Consent needed for continuous monitoring of user state

- Medical diagnosis implications (implying user has fall risk)

- Elderly user monitoring may have paternalism concerns

- EMS dispatch cost liability for false alarms

- Device movement during activities (e.g., falling while playing sports) triggering false alerts

# 10. Privacy Protections

- Accelerometer data buffered locally, not streamed to server

- Activity labels generated on-device using on-device ML model

- Fall event data includes aggregates only (impact magnitude, duration), not raw traces

- Raw sensor traces deleted 24h after event if no alert sent

- EMS notifications anonymized with location only, no user identity

- Elevation data sourced from terrain DB, not from GPS altitude (less precise)

- User can manually dismiss fall alerts to prevent cascading notifications

- Fall detection can be disabled by user at any time

- Activity recognition not shared with health apps or insurers