Atlas — Global Source Layers

Buildings (Global) · Roads · OSM Infrastructure · Satellite Basemap

Status: Fully Specced

Global source layers are extracted from worldwide datasets before US-specific refinement. These form the baseline for areas without LiDAR or high-resolution imagery, and provide the vector skeleton that national refinement layers are fused onto.

# 1. Building Footprints (Global)

Three independent global building datasets are maintained as separate estimation layers with confidence scores. They are NOT merged — each feeds the fused building layer with its own confidence weight.

## 1.1 Google Open Buildings

| Attribute | Detail |
| --- | --- |
| Layer name | buildings_google |
| Type | Vector polygons (GeoParquet) |
| Source | Google Open Buildings v3 (ML detection from satellite imagery) |
| Coverage | Africa, Asia, Americas — strong global coverage |
| Attributes | Footprint polygon, area_m², confidence_score (0–1), plus_code |
| Key uses | Primary global building layer. Fused with Microsoft and Overture. Where LiDAR unavailable, this is the building source. |
| Limitations | No height, no construction type, no material. Satellite-detected only — misses buildings under dense canopy. |
| User-facing? | No — feeds fused buildings layer |

## 1.2 Microsoft Building Footprints

| Attribute | Detail |
| --- | --- |
| Layer name | buildings_microsoft |
| Type | Vector polygons (GeoParquet) |
| Source | Microsoft GlobalMLBuildingFootprints (ML from Bing imagery) |
| Coverage | Global, strong in US/Europe/Australia |
| Attributes | Footprint polygon, height (where available from DSM), confidence |
| Key uses | Independent estimate for fusion. Used extensively in ELMFIRE WUI research (Gollner lab uses Microsoft footprints for structure locations in fire spread modeling). |
| User-facing? | No — feeds fused buildings layer |

## 1.3 Overture Maps Buildings

| Attribute | Detail |
| --- | --- |
| Layer name | buildings_overture |
| Type | Vector polygons (GeoParquet) |
| Source | Overture Maps Foundation (aggregates OSM + commercial sources) |
| Attributes | Footprint, height, floors (where available), building use type, name |
| Key uses | Highest attribute richness globally. Building use type (residential/commercial/industrial) feeds vulnerability classification. Height from multiple sources. |
| User-facing? | No — feeds fused buildings layer |

# 2. Roads (Global)

## 2.1 OpenStreetMap Road Network

| Attribute | Detail |
| --- | --- |
| Layer name | roads_osm |
| Type | Vector lines (GeoParquet) |
| Source | OpenStreetMap |
| Attributes | highway class, name, surface type, lanes, max_speed, access restrictions, bridge/tunnel flags, oneway, width (where tagged) |
| Key uses | Primary road network for Exodus routing. Edge weights: travel time, hazard exposure, road type. Evacuation route planning. Road closure overlay. Bridge identification for flood/earthquake damage flagging. |
| Limitations | Surface type often untagged. Width rarely specified. Quality varies by region. |
| US supplement | LiDAR-measured road widths, DOT data, Mapillary street view improve attribute completeness |
| User-facing? | 🗺️ YES — Roads layer (from enriched road product) |

# 3. OSM Infrastructure and Points of Interest

## 3.1 Critical Facilities

| Attribute | Detail |
| --- | --- |
| Layer name | critical_facilities |
| Type | Vector points + polygons |
| Source | OpenStreetMap + HIFLD (Homeland Infrastructure Foundation-Level Data, US) |
| Categories | Hospitals, fire stations, police stations, emergency management offices, EOCs, National Guard armories, Coast Guard stations, Red Cross chapters |
| Attributes | Name, facility type, address, phone, beds (hospitals), 24hr status, backup power status (where known) |
| Key uses | Emergency routing: "nearest hospital with trauma capability." Resource pre-positioning for EM. Shelter-of-last-resort identification. Evacuation staging. |
| User-facing? | 🗺️ YES — Critical Facilities, toggleable |

## 3.2 Evacuation Shelters

| Attribute | Detail |
| --- | --- |
| Layer name | evacuation_shelters |
| Type | Vector points |
| Source | FEMA National Shelter System (NSS); Red Cross; OSM; state emergency management shelter registries |
| Attributes | Name, address, capacity, pet-friendly status, accessible (ADA), medical support level, generator, current activation status (real-time feed during events) |
| Key uses | Core Exodus routing destination: route user to nearest appropriate shelter. Pet-friendly flag for users with animals. ADA flag for mobility needs. Real-time capacity feed to avoid routing to full shelters. |
| User-facing? | 🗺️ YES — Evacuation Shelters layer, always visible during active events |

## 3.3 Tornado Shelter Registry

| Attribute | Detail |
| --- | --- |
| Layer name | tornado_shelters |
| Type | Vector points |
| Source | FEMA safe room database; user-contributed; OSM; extended to include non-standard structures |
| Structure types | FEMA-rated safe rooms, bank vaults, walk-in freezers (discussed explicitly — commercial refrigerators are tornado-rated by construction), concrete basements, interior concrete rooms |
| Key uses | Tornado shelter routing: user in tornado warning → route to nearest rated shelter. Walk-in freezers identified as valid tornado shelters — Beacon extends this beyond formal registries. |
| User-facing? | 🗺️ YES — Tornado Shelters, active during warnings |

## 3.4 Vulnerable Population Facilities

| Attribute | Detail |
| --- | --- |
| Layer name | vulnerable_population_facilities |
| Type | Vector points |
| Source | CMS (nursing homes, dialysis centers, hospitals); state licensing databases; OSM; HIFLD |
| Categories | Nursing homes, assisted living, dialysis centers, hospitals (with bed count), schools, daycares, group homes, homeless shelters, prisons, college dormitories, disability services |
| Key uses | The consolidated layer emergency managers have always wanted but currently scattered across 6+ state agencies. Pre-event: identify facilities needing evacuation assistance. During event: prioritize rescue/evacuation resources. "There is a dialysis center with 40 patients in your evacuation zone." |
| User-facing? | 🗺️ YES — Vulnerable Facilities, EM view + public toggle |

## 3.5 Cooling Centers / Warming Centers

| Attribute | Detail |
| --- | --- |
| Layer name | thermal_relief_centers |
| Type | Vector points |
| Source | Municipal/county activation feeds + OSM + FEMA + American Red Cross |
| Attributes | Name, address, capacity, hours (real-time activation status), accessible, pet-friendly, transit accessible |
| Key uses | Extreme heat: route vulnerable users to nearest open cooling center. Extreme cold: route to warming center. Real-time activation status critical — many only open during declared events. |
| User-facing? | 🗺️ YES — Cooling/Warming Centers, active during heat/cold events |

## 3.6 Helicopter Landing Zones

| Attribute | Detail |
| --- | --- |
| Layer name | helicopter_lz |
| Type | Vector polygons |
| Source | FAA helipad registry; LiDAR-derived cleared area detection; NAIP open space classification; known hospital helipads |
| Requirements | Flat (<5° slope), clear of obstacles (nDSM), minimum 30m × 30m, paved surface preferred but not required |
| Key uses | Mass casualty routing: "nearest LZ for medical helicopter." Wildfire air operations: "closest clear LZ to this address." Search and rescue coordination in backcountry. |
| User-facing? | 🗺️ YES — Landing Zones, EM + responder view |

## 3.7 Fuel Stations

| Attribute | Detail |
| --- | --- |
| Layer name | fuel_stations |
| Type | Vector points |
| Source | OSM + commercial POI data |
| Attributes | Fuel type (gasoline, diesel, EV charging), brand, 24hr status, backup generator (where known) |
| Key uses | Evacuation routing: fuel along route. "Your vehicle may not reach the shelter without refueling — gas station at X is 8 miles." EV routing: charging station availability. |
| User-facing? | 🗺️ YES — Fuel Stations, evacuation mode |

## 3.8 Bridges

| Attribute | Detail |
| --- | --- |
| Layer name | bridges |
| Type | Vector points + spans |
| Source | FHWA National Bridge Inventory; OSM bridge tags |
| Attributes | Structural condition rating (0–9), load limit (tons), scour vulnerability, year built, material type, bridge length, lanes, waterway clearance |
| Key uses | Routing weight: structurally deficient bridges get penalty or exclusion. Post-earthquake: bridges flagged for avoidance. Flood: scour vulnerability at high water. Vehicle weight routing for heavy vehicles. |
| User-facing? | No — routing engine input |

## 3.9 Dams and Reservoirs

| Attribute | Detail |
| --- | --- |
| Layer name | dams_reservoirs |
| Type | Vector points + polygons |
| Source | USACE National Inventory of Dams (~90,000 dams); OSM |
| Attributes | Dam name, hazard classification (high/significant/low), height, reservoir capacity, dam type, owner, EAP (Emergency Action Plan) availability, downstream population |
| Key uses | Dam failure module: high-hazard dams get pre-computed downstream inundation scenarios. Reservoir level monitoring. "Oroville Dam breach at current reservoir level → your address inundated in 47 minutes." |
| User-facing? | 🗺️ YES — Dams (high-hazard class), toggleable |

# 4. Power Infrastructure (Global/OSM)

## 4.1 Power Lines

| Attribute | Detail |
| --- | --- |
| Layer name | power_lines_osm |
| Type | Vector lines |
| Source | OSM power lines; utility territory data; supplemented by LiDAR corridor detection |
| Attributes | Voltage class (transmission/subtransmission/distribution), line type (overhead/underground), operator, span geometry |
| Key uses | PSPS (Public Safety Power Shutoff) zone definition. Wildfire ignition risk from powerlines. Post-flood: electrocution risk from downed lines + flood water (Hurricane Katrina scenario). Exodus routing: avoid roads under downed lines. LiDAR confirms clearance heights and detects vegetation encroachment. |
| Note | LiDAR detection of power lines discussed — complex due to scan geometry. OSM + utility data preferred for line corridors; LiDAR for clearance height verification and vegetation encroachment detection. |
| User-facing? | 🗺️ YES — Power Lines, EM toggle; Electrocution Risk overlay during flood events |

## 4.2 Utility Territories

| Attribute | Detail |
| --- | --- |
| Layer name | utility_territories |
| Type | Vector polygons |
| Source | EIA electric service territory data; state PUC filings |
| Attributes | Utility name, type (IOU/co-op/municipal), contact info, PSPS history (CA IOUs) |
| Key uses | PSPS coordination: identify which utility controls power for a given address. Alert integration: PG&E/SCE/SDG&E have public PSPS notification feeds. "Power managed by PG&E — PSPS possible within 12 hours." |
| User-facing? | No — alert logic input |