Atlas — US LiDAR (3DEP) Extraction Layers

USGS 3DEP Point Cloud Processing · ~40 Derived Layers

Status: Fully Specced

USGS 3DEP (3D Elevation Program) provides raw LiDAR point cloud data covering most of the US at QL2 quality (2–4 pts/m²) with higher quality QL1 in many urban areas (8+ pts/m²). Processing runs as a 4-stage pipeline per grid cell: classification → rasterization → object extraction → spatial analysis.

Key architectural decisions:

- Each source produces independent estimation layers — LiDAR, NAIP, and Mapillary each generate separate building footprint layers with separate confidence scores. They are NOT merged at this stage.

- 50-meter buffer around each grid cell is cached to reconcile edge artifacts with neighboring cells.

- Detection confidence rasters track where the LiDAR scanner could and could not see (shadow zones) — absence of detection ≠ absence of object.

- ALSO (ray-aware occupancy network) used as foundation model — learns aerial LiDAR physics from unlabeled 3DEP data, distinguishing confirmed empty space from unscanned shadow zones.

# Pass 1 — Physics/Geometry (PDAL-based)

## Terrain Rasters (6)

| Layer | Type | Description | Key Uses |
| --- | --- | --- | --- |
| lidar_dem | Raster Float32 | Bare-earth DEM. Ground-classified points only. Higher resolution than Copernicus (down to 1m). Overrides global DEM in fusion. | All terrain derivatives. HAND recomputation at higher res. |
| lidar_dsm | Raster Float32 | Digital Surface Model — includes everything (buildings, trees, structures). Cannot be obtained from Copernicus (bare-earth only). | nDSM computation. Building height. Vegetation height. Viewshed. |
| lidar_slope | Raster Float32 | Slope derived from LiDAR DEM. Higher resolution than Copernicus-derived slope. | Fine-scale vehicle passability. Avalanche terrain (>30°). Fire spread local acceleration. |
| lidar_aspect | Raster Float32 | Aspect at LiDAR resolution. | Fine-scale fuel moisture. Solar exposure per parcel. |
| lidar_roughness | Raster Float32 | Surface roughness / elevation variance at LiDAR resolution. | Fine-scale vehicle passability. Micro-terrain for ember deposition. |
| lidar_depression_depth | Raster Float32 | Depression/sink depth at LiDAR resolution. | Urban pooling locations. Flash flood micro-routing. |

## Vegetation Rasters (8)

| Layer | Type | Description | Key Uses |
| --- | --- | --- | --- |
| lidar_chm | Raster Float32 | Canopy Height Model (DSM − DEM). Height of vegetation above ground in meters. | Crown fire potential. Fuel structure for ELMFIRE. Ember generation height. |
| lidar_canopy_cover | Raster Float32 (0–1) | Canopy cover fraction — proportion of ground covered by canopy. | Fire crown potential. Shade calculation. Fuel density. |
| lidar_veg_0_0.5m | Raster Float32 (pts/m²) | Point density in 0–0.5m stratum — surface fuels (litter, duff). | Fire surface fuel load. Flash flood infiltration. |
| lidar_veg_0.5_1m | Raster Float32 (pts/m²) | Point density in 0.5–1m stratum — short shrubs, grass. | Ladder fuels lower tier. |
| lidar_veg_1_3m | Raster Float32 (pts/m²) | Point density in 1–3m stratum — shrubs, young trees. CRITICAL for WUI fire modeling. | Ladder fuels. Bridge fuel between ground and canopy. WUI ignition pathway. |
| lidar_veg_3_10m | Raster Float32 (pts/m²) | Point density in 3–10m stratum — midstory trees. | Ladder fuel upper tier. Ember production height. |
| lidar_veg_10m_plus | Raster Float32 (pts/m²) | Point density above 10m — overstory canopy. | Crown fire. Large ember production. |
| lidar_understory_density | Raster Float32 | Vegetation density below canopy. | Fire penetration under canopy. Ladder fuel continuity. |

## Surface Rasters (2)

| Layer | Type | Description | Key Uses |
| --- | --- | --- | --- |
| lidar_intensity | Raster UInt16 | Mean return intensity, range-normalized. Bright = high reflectance (pavement, metal). Dark = low reflectance (water, vegetation). | Surface material classification. Road surface type. Metal roof detection. Water body identification. |
| lidar_surface_texture | Raster Float32 | Ground point variance — rough = rocks/debris, smooth = pavement/water. | Vehicle passability. Road surface type. Off-road terrain characterization. |

## Quality / Coverage Rasters (2)

| Layer | Type | Description | Key Uses |
| --- | --- | --- | --- |
| lidar_point_density | Raster Float32 (pts/m²) | Raw point return density per grid cell. Shows where scanner had good coverage. | Data quality assessment. Confidence weighting in fusion. Coverage tracker. |
| lidar_shadow_coverage | Raster Float32 (0–1) | CRITICAL: For each cell, what fraction of the space was actually scanned by a LiDAR ray? Derived from ray geometry. 0 = complete shadow zone (never illuminated). 1 = fully observed. This is NOT the same as point density — a cell can have zero points but be in a confirmed empty space (ray passed through at ground level). | Absence of detection ≠ absence of object. Confidence weighting in all downstream layers. Multi-source fusion: where LiDAR has shadow zones, weight Mapillary or NAIP more heavily. |

# Pass 2 — ML-Based Object Extraction

## Buildings (LiDAR-Derived)

| Layer | Type | Description | Key Uses |
| --- | --- | --- | --- |
| lidar_building_footprints | Vector GeoParquet | Building footprints from LiDAR point clustering. Attributes: height_mean, height_max, height_std, area_m², floor_count_estimate, structure-to-structure distances, nearest_vegetation_distance_m. | Flood first-floor elevation. Fire vulnerability: structure distance to nearest vegetation. Building height for tsunami vertical evacuation assessment. |
| lidar_building_advanced | Vector GeoParquet | Second-stage analysis of building point subsets (PointNet++). Roof type classification: flat/gable/hip/complex. Roof material estimate. Solar panel detection. Helicopter landing assessment (flat roof + sufficient area + no obstructions). | Fire vulnerability (flat roof = ember accumulation). Solar panel → metal/non-combustible roof hint. Helicopter LZ pre-screening. |

## Barriers

| Layer | Type | Description | Key Uses |
| --- | --- | --- | --- |
| lidar_barriers | Vector GeoParquet (polylines) | Fence, wall, and guardrail polylines extracted from linear vertical features in ML-classified point cloud. Attributes: height_m, length_m, type (fence vs. wall by thickness), confidence. | CRITICAL for WUI fire modeling: wooden fences are bridge fuels enabling structure-to-structure spread. Exodus routing: barrier traversability. Passability layer: fences are lateral boundaries. Combined with Mapillary CV for type confirmation. |

## Vegetation Detail

| Layer | Type | Description | Key Uses |
| --- | --- | --- | --- |
| lidar_vegetation_detail | Vector GeoParquet | Individual tree detections: trunk location (2D), height (m), crown diameter (m), species group classification (conifer/deciduous/shrub). Vegetation-to-structure distances. Ladder fuel zone polygons (areas where 1–3m stratum creates continuous fuel connection from ground to canopy). | Parcel-level bridge fuel graph for WUI fire modeling. Individual trees for fine-scale ember production. Ladder fuel zones for crown fire ignition risk. ELMFIRE currently uses 30m LANDFIRE — Beacon adds individual tree resolution. |

## Surface Features

| Layer | Type | Description | Key Uses |
| --- | --- | --- | --- |
| lidar_surface_features | Vector GeoParquet | Power line poles and spans (confirmed by correlated point patterns), curbs, road widths (measured from LiDAR cross-sections including shoulder), driveways, parking areas. | Road width for vehicle passability. Driveway detection for off-road routing connections. Power line clearance height verification. Curb detection for urban flood micro-routing. |

## Cylindrical Objects (Tanks/Silos)

| Layer | Type | Description | Key Uses |
| --- | --- | --- | --- |
| lidar_cylinders | Vector GeoParquet (points) | Cylindrical object detections: centroid, radius, height, orientation, confidence. Candidates for propane tanks, water tanks, grain silos, industrial storage. | Explosion Hazard Zones: cylindrical objects near structures → propane/fuel tank candidate → assign hazard radius. Cross-referenced with Mapillary CV for visual confirmation. |

## Cleared Areas

| Layer | Type | Description | Key Uses |
| --- | --- | --- | --- |
| lidar_cleared_areas | Vector polygons | Areas with no canopy and low nDSM — fields, clearings, open lots, meadows. Attributes: area_m², surrounding vegetation stats, slope. | Helicopter landing zone pre-screening. Fire shelter-in-place areas ("defensible large parking lot" identification). Wildfire "in the black" zone tracking. |

## Soil Estimate (Low Confidence)

| Layer | Type | Description | Key Uses |
| --- | --- | --- | --- |
| lidar_soil_estimate | Raster | Rough soil classification from LiDAR intensity + texture + slope + vegetation context. Classes: rocky, sandy, clay, organic. Explicitly low confidence. | Supplementary input where SSURGO soil data unavailable. Passability: rocky/sandy affect vehicle traction. Flood infiltration rate rough estimate. |

# Viewshed / Line of Sight

| Layer | Type | Description | Key Uses |
| --- | --- | --- | --- |
| lidar_viewshed | Raster (on-demand) | What can be seen from a given point, accounting for buildings and vegetation from LiDAR DSM. | Fire spotting: where could an observer see fire from? Communication planning: where does Hermes mesh radio have line-of-sight? Search and rescue: observer positioning. |

# Spatial Relationship Layers

| Layer | Description |
| --- | --- |
| structure_to_structure_distances | Distance matrix between building centroids. Used in WUI fire spread model: close structures = high risk of structure-to-structure ignition via embers/radiation. |
| structure_to_vegetation_distances | Distance from each building to nearest vegetation by height class. Defensible space compliance proxy. Ember transport distance. |
| wui_boundary | Wildland-Urban Interface boundary: where residential development meets or intermingles with wildland vegetation. Derived from structure density + vegetation density. LANDFIRE WUI layer as reference. |
| ladder_fuel_zones | Polygons where 1–3m vegetation stratum creates continuous fuel pathway from ground to canopy. Critical for crown fire initiation. Absent from LANDFIRE 30m grid. |

# nDSM (Normalized Digital Surface Model)

| Attribute | Detail |
| --- | --- |
| Layer name | ndsm |
| Computation | DSM − DEM = nDSM. Shows height of everything above ground. |
| Key uses | Building detection (nDSM > 2m, locally flat). Vegetation height. Object height. Flood first-floor elevation estimate. Routing obstacle mapping. |
| Explicitly discussed as | "The LiDAR-derived raster that shows all objects above terrain" — primary obstacle layer for drivability assessment. |

# ML Model Architecture for LiDAR Processing

Four-tier pipeline designed specifically for aerial LiDAR physics:

| Tier | Model | Role |
| --- | --- | --- |
| Tier 1 | PDAL (physics/geometry) | Ground classification, DEM/DSM derivation, shadow zone computation, synthetic facade extrusion from building footprints for pseudo street-level geometry. |
| Tier 2 | Specialist models (RandLA-Net, PointNet++) | Building segmentation, roof type classification, barrier extraction, vegetation structure classification, surface/pavement classification. |
| Tier 3 | ALSO (ray-aware occupancy network) | Self-supervised pre-training on raw 3DEP data. Learns aerial LiDAR acquisition physics — distinguishes confirmed empty space from shadow zones. Output: 512-dim per-point embeddings that encode geometric understanding AND sensor awareness. |
| Tier 4 | LeJEPA (multi-modal fusion) | Cross-modal prediction tasks between LiDAR, satellite, and street view. Output: unified embeddings feeding all downstream task heads (fire fuel, routing, structure vulnerability, flood risk). |