Atlas/Exodus — Vehicle Database and Routing Layers

Vehicle Passability · Thermal Thresholds · Routing Graph Architecture

Status: Fully Specced

The vehicle database was designed as a 30-file, 24-table architecture supporting both a reference database of standard vehicles and a user-input system with conservative fallback estimates. It directly feeds the Exodus routing engine's passability calculations.

# Core Philosophy

- Conservative fallback: unknown vehicle characteristics default to worst-case values for safety.

- User input overrides: user-specified vehicle specs unlock better routing options.

- CV estimation: computer vision can estimate vehicle class from photo for new/unknown models.

- Pre-computed passability matrices: avoid real-time physics calculations during emergency routing.

# Vehicle Reference Database

| Table | Key Attributes |
| --- | --- |
| manufacturers | make, country_of_origin, vehicle_classes_produced |
| vehicle_specs | make, model, year_range, class (sedan/SUV/truck/van/motorcycle), ground_clearance_mm, length_m, width_m, height_m, turning_radius_m, weight_kg, drivetrain (2WD/4WD/AWD) |
| tire_specs | linked to vehicle_specs, tire_width_mm, tire_height_mm, load_index, all_season (bool), mud_rated (bool) |
| terrain_passability | vehicle_id, surface_type, slope_max_degrees, passable (bool), speed_modifier (0–1), notes |
| thermal_thresholds | vehicle_id, undercarriage_max_temp_C, tire_max_temp_C, engine_max_ambient_C, time_to_damage_at_threshold_min |
| flood_thresholds | vehicle_id, intake_height_mm (engine hydro-lock risk), electrical_floor_height_mm, wheel_well_flood_height_mm |
| electrical_hazard | vehicle_id, safe_proximity_downed_line_m (voltage class dependent), hybrid_ev_flag (additional risk in flood) |

# Thermal Thresholds (Wildfire Routing)

Wildfire creates unique routing constraints. Explicitly discussed:

- Burned-over grassland may be passable within minutes after flame front if fuel load was light — tires can withstand brief exposure to residual ground heat.

- Heavy timber smoldering: tires at risk for 2–6 hours after flame front passage.

- Tire melting points: ~260°C for rubber. Ground temps after grass fire return to passable range quickly; after heavy fuel fire, can remain dangerous for hours.

- Undercarriage clearance: burning debris on road surface vs. vehicle clearance height.

| Fuel Type | Post-Flame Passability | Tier |
| --- | --- | --- |
| Dry grass (Model 1–3) | 15–30 minutes after front | Potentially passable quickly |
| Brush/shrub (Model 4–7) | 30–90 minutes | Moderate wait |
| Light timber (Model 8–9) | 2–4 hours | Extended wait |
| Heavy timber (Model 10) | 4–6+ hours | Avoid until cleared |

# Flood Passability

- Sedan engine intake height: ~400–600mm. Above this = engine hydro-lock.

- Flow velocity matters more than depth: fast-moving shallow water is more dangerous than deep still water.

- EV/hybrid: additional electrical risk in flood water — lower threshold.

- Bridge scour: bridges can fail as flood rises even if road surface above water.

# Routing Graph Architecture

| Component | Description |
| --- | --- |
| Road network graph | OSM roads as directed edge graph. Edges enriched with: travel time, road class, surface type, width, slope profile, hazard exposure tags. |
| Off-road graph extension | Parking lots, driveways, fields, road shoulders added as traversable edges with surface type and slope. Road-to-off-road connection points as graph nodes. |
| Dynamic edge closure | Real-time closure events (fire, flood, road report) update edge passability flag. Routing engine re-runs on closure. |
| Vehicle-specific filtering | Pre-computed passability matrices indexed by vehicle class × surface type × slope. Lookup takes <1ms per edge evaluation. |
| Hazard exposure weighting | Edges in fire corridor, flood-prone road, landslide path, ice-prone bridge get penalty multiplier. Not binary closure — still routable if no better option, but deprioritized. |
| A* routing | A* algorithm with f(n) = g(n) + h(n). h(n) = straight-line distance × speed_factor. More efficient than Dijkstra — explores toward goal rather than all directions. Heuristic must be admissible (never overestimates). |
| Multi-objective routing | Can optimize for: fastest, safest (minimize hazard exposure), least congested, or user preference weighting of these. |

# User Vehicle Input System

| Input Mode | Description |
| --- | --- |
| Manual spec entry | User selects make/model/year → pre-populated from reference database. |
| Quick class selection | User selects "sedan / SUV / truck / motorcycle / no vehicle (on foot)" → conservative class defaults applied. |
| Photo estimation (CV) | User photos vehicle → CV classifies make/model estimate → lookup reference database. Triage tool for new/unknown models. |
| Unknown vehicle fallback | If vehicle unknown: default to sedan specs (worst 2WD case). Conservative philosophy: underestimates capability rather than overestimates. |

# Satellite Communication for Routing (Hermes Integration)

Routing protocol bandwidth constraints discussed in context of satellite SOS and LoRa:

- Starlink: 100–200 Mbps (full routing possible)

- Iridium (satellite phone): 2.4 kbps (text-only, compressed BSP packets only)

- LoRa mesh: ~250 bps effective (minimal compressed payload)

- Apple/Garmin satellite SOS: 140 byte maximum message

- Solution: pre-compressed "seed" data (20–50 bytes: hazard type + severity + direction) that devices expand using pre-loaded terrain data + on-device model. 20–100× effective compression ratio.