Tsunami

Jupiter Hazard Module — Beacon System Reference

Status: Fully Specced

# Overview

Tsunami is the most tractable of Beacon's early hazard modules — discrete trigger, deterministic propagation physics, terrain-driven vulnerability. Two complete specification documents exist: launch system and future model architecture.

Strategic decision: Beacon's value is last-mile delivery of existing official forecasts to individuals at their exact location + routing to high ground. Not competing with PTWC on forecast accuracy.

# The Core Gap Beacon Fills

## Far-Field Events (e.g., Alaska, Chile → West Coast)

- Official forecasts (PTWC) available 15–60+ minutes before arrival

- Gap: translating a regional polygon warning into "you specifically, at your address, have X minutes to reach Y elevation"

- Beacon's role: take PTWC forecast → user's exact GPS → nearest high ground → turn-by-turn routing

## Near-Field Events (Cascadia Subduction Zone)

- Tsunami arrives in ~15 minutes at parts of Oregon/Washington coast

- Official forecast cycle takes 15–30 minutes — arrives AFTER the wave

- This is the real gap. Beacon cannot wait for PTWC for Cascadia.

- Solution: ShakeAlert integration — when CSZ rupture detected, trigger immediate vertical evacuation alert based on pre-computed scenarios and magnitude estimate

- No time for routing — "move uphill NOW, maximum elevation, fastest path"

# Launch System Architecture

## Detection Layer

| Source | What It Provides | Latency |
| --- | --- | --- |
| USGS Earthquake API | Magnitude, location, depth within seconds of event | <60 seconds |
| ShakeAlert | P-wave detection → S-wave warning (seconds of lead time) | Near-real-time |
| DART buoys | Open-ocean wave detection confirming tsunami generation | 15–30 min |
| PTWC / NWS alerts | Official watch/warning/advisory | 15–30 min |
| S-net / DONET | Japanese offshore seismometer/pressure sensor networks | Minutes |

## Alert Triggering Logic

| Magnitude | Depth | Location | Action |
| --- | --- | --- | --- |
| M7.5–8.0 | <70km | Subduction zone | Alert users below 15m within 1km coast |
| M8.0–8.5 | <70km | Subduction zone | Alert below 20m within 2km coast |
| M8.5+ | <70km | Full margin | Alert below 30m within 3km; vertical evac if travel time > ETA |
| Any M7.5+ | Near CSZ | Cascadia | Immediate: no PTWC wait, pre-computed scenario activation |

USGS finite fault products give rupture extent within minutes for large events — used to scale response.

## Inundation Mapping

- Pre-computed NOAA inundation zones for catalogued scenarios at US coastal locations

- HAND-derived coastal vulnerability for uncatalogued areas (fast approximation)

- Post-earthquake: co-seismic deformation applied to coastal elevation model (land uplift/subsidence changes effective height above sea level)

- Okada model for co-seismic deformation from fault parameters

- Known faults (Cascadia, Nankai) have pre-computed uplift/subsidence scenarios — triggered on event, no real-time calculation

## Routing to Safety

- Atlas elevation data: user's exact elevation above sea level

- Pre-computed vertical evacuation routes to high ground or tsunami refuge structures

- Vertical evacuation buildings: tagged in Atlas (reinforced concrete, sufficient height)

- ETA calculation: wave speed (√(g×depth) in shallow water) → arrival time at coastline → user time budget

- If travel time to high ground > available time: route to nearest vertical evac structure

- Multi-wave problem: alert users NOT to return after first wave — subsequent waves often larger

# Future Model Architecture (Long-Term)

## Phase 1 — Coarse Solver + Neural Corrector

- Run fast shallow-water equation solver (coarse resolution)

- Neural network learns systematic errors vs. high-fidelity GeoClaw runs

- Analogous to weather model post-processing

## Phase 2 — Differentiable Solver with Learned Subgrid

- Physics solver made differentiable (JAX or similar)

- Subgrid processes (wetting/drying, Manning friction variations, bathymetry uncertainty) learned from data

- End-to-end trainable with observational data

## Phase 3 — Graph Network Simulator

- Learned simulation on irregular mesh (nodes = coastal terrain points)

- Analogous to NeuralGCM trajectory in weather modeling

- Directly trainable from DART buoy observations + runup measurements

- This is the convergence target for the field

## Data Collection (Starting Immediately)

Even before sophisticated model exists, collect ground truth now:

- DART buoy waveform histories

- Historical tide gauge records during tsunami events

- Post-event runup surveys (NOAA/USGS field surveys)

- High-resolution bathymetry (NOAA Coastal Relief Model)

- Co-seismic deformation records

- Pair all data: seismic parameters → observed wave → runup

# Atlas Layer Integration

- Coastal elevation model (Copernicus DEM + co-seismic deformation adjustment)

- Pre-computed NOAA inundation zone polygons

- DART buoy locations

- Vertical evacuation structure registry

- Tsunami runup history (historical events)

- Fault lines with rupture scenario metadata (USGS Slab2.0)

- Bathymetry for offshore wave speed calculation

# BSP Encoding (Hermes)

- D30: Tsunami generated

- D31/D32/D33: Tsunami ETA >3hr / 1-3hr / <1hr

- D34–D36: Arriving / first wave / subsequent waves

- D37: All clear

- C5A/C5B/C5C: Warning / watch / advisory