Avalanche

Jupiter Hazard Module — Beacon System Reference

Status: Fully Specced

# Overview

Avalanche is a fully specced Jupiter module with two complete specification documents: the user-facing client system and the internal employee beacon_dev system for model training and validation. Primary beachhead: Beacon Alpine (ski resorts, backcountry, mountaineering).

# User-Facing System

## Alert Delivery

- Ingests official avalanche forecasts (NWAC, CAIC, UAC, etc.) and delivers at user's exact location

- User sees: their current terrain's avalanche danger rating, aspect/elevation-specific risk, current weak layer concerns

- Beacon Alpine: in-resort users get slope-specific danger as they navigate terrain

- Backcountry: route planning integrates avalanche terrain rating (ATES: simple/challenging/complex)

## Terrain Classification

- Avalanche terrain polygons overlaid on Atlas map

- ATES classification per terrain zone

- Aspect + elevation band risk matrix (standard avalanche forecast format)

- Slope angle layer from USGS 3DEP LiDAR — slope > 30° flagged as avalanche terrain

- Runout zones pre-computed from terrain modeling

## Search & Rescue Integration

- Ski patrol draws affected zone polygon on map — system identifies all phones within it

- Location data of potential burial victims (last known position before event)

- Mesh networking (Hermes) relays beacon signals in infrastructure-down scenarios

- BLE ranging for relative positioning of rescuers and victim search areas

# Internal System: beacon_dev

## Model Training Pipeline

- SNOWPACK model as physics-based reference for weak layer development, snow settlement, temperature gradient

- Weather input: HRRR/GFS for temperature, precipitation, wind

- Historical avalanche occurrence database: location, date, aspect, elevation, trigger, size

- Automatic detection: seismic sensors, infrasound, time-lapse cameras on known paths

## Professional Feedback Loop

Key differentiator: ski patrol and certified avalanche forecasters can flag model disagreements:

- Forecaster flags: "model said low danger, we saw natural activity on NE aspects at 8,000ft"

- Patrol flags: "we triggered a large avalanche on a slope model rated moderate"

- Each flag is a weighted training signal — professional credentials = higher weight

- Disagreement dashboard shows model vs. field observations side-by-side

- Feedback accumulates into retraining dataset

## Data Labeling System

- Time-lapse camera review UI for labeling avalanche events

- Seismic event annotation for discriminating avalanche vs. other signals

- Field observation import from Mountain Hub, NWAC Observer, state programs

- Confidence scoring per label by source credibility

## Validation Framework

- Backtesting against historical events with held-out data

- Spatial cross-validation: train on one range, test on another

- Comparison against official forecasts as baseline

- Confusion matrix by danger rating: false lows are safety-critical

# Data Sources

| Source | What It Provides |
| --- | --- |
| NWAC / CAIC / UAC / regional centers | Official avalanche forecasts by zone and elevation band |
| SNOWPACK model | Physics-based snow stratigraphy and weak layer evolution |
| HRRR / GFS | Wind, temperature, precipitation for snowpack forcing |
| USGS 3DEP LiDAR | Slope angle, aspect, terrain curvature for terrain classification |
| SNOTEL stations | Snow water equivalent, temperature, precipitation at fixed sites |
| USGS seismic network | Seismic signals for automatic avalanche detection |
| Field observers / ski patrol | Ground-truth observations via professional feedback loop |
| Time-lapse cameras | Visual confirmation of avalanche activity on instrumented paths |

# Beacon Alpine Context

Mountain safety was identified as a potential beachhead market:

- Ski resorts: paid integration for patrol dispatch coordination, guest safety alerting, terrain closure management

- Guide services: expedition planning with real-time hazard integration

- Proves core technology (mesh networking, location-based alerting) in contained, seasonal environment

- Revenue funds broader disaster response development

- Cold weather battery concerns → argument for dedicated low-power mesh hardware (LoRa beacons)

# BSP Encoding (Hermes)

- C3D: Avalanche occurred

- C3E: Avalanche warning

- Observation codes: avalanche sighting, debris field, burial, safe zone confirmed