Landslide / Rockslide / Debris Flow

Jupiter Hazard Module — Beacon System Reference

Status: Planned

# Overview

Landslides, rockslides, and debris flows share terrain-driven susceptibility but differ in trigger mechanisms and runout behavior. Beacon's approach: pre-computed susceptibility combined with real-time trigger monitoring (rainfall intensity, earthquake shaking, wildfire burn scars).

# Sub-Types

| Type | Trigger | Speed | Prediction Difficulty |
| --- | --- | --- | --- |
| Shallow landslide | Intense rainfall, saturation | Fast | Medium — rainfall threshold known |
| Deep-seated landslide | Long-term saturation, loading | Slow | High — precursory movement detectable |
| Rockslide / rockfall | Freeze-thaw, seismic, undercutting | Very fast | Low — fracture state rarely known |
| Debris flow | Post-fire rainfall + channel geometry | Very fast | Medium — burn scar + rainfall threshold |
| Lahar | Volcanic eruption + snowmelt | Very fast | Medium — volcanic trigger known |
| Mudslide | Saturation of fine-grained soils | Moderate | Medium |

# Post-Fire Debris Flow — Priority Case

Post-wildfire debris flows are the most predictable and highest-consequence landslide type for Beacon:

- Burn scars destroy soil cohesion and hydrophobic layer forms on surface

- Any significant rainfall on a recent burn scar triggers debris flow risk

- Channels in steep terrain concentrate and accelerate flows

- USGS publishes post-fire debris flow hazard assessments for major fires within days

- Rainfall threshold models: NOAA rainfall forecasts + USGS thresholds → automated alert

# Data Sources

| Source | What It Provides |
| --- | --- |
| USGS Landslide Hazards Program | Susceptibility maps, post-fire assessments, historical inventory |
| NASA Global Landslide Catalog | Global historical event database |
| USGS 3DEP LiDAR | Slope, aspect, curvature, drainage basin geometry |
| LANDFIRE disturbance layer | Recent burn scars for post-fire debris flow risk |
| MRMS / rain gauges | Real-time rainfall intensity for threshold comparison |
| USGS seismic network | Earthquake trigger detection |
| InSAR (Sentinel-1) | Pre-failure slope deformation detection for deep-seated slides |

# Atlas Layer Integration

- Landslide susceptibility zones (USGS)

- Landslide runout zones (modeled from terrain + inventory)

- Burn scar polygons (from NASA FIRMS + NIFC, updated post-fire)

- Debris flow channel networks in high-susceptibility areas

- Historical landslide inventory points

# Alert Logic

- Rainfall threshold exceeded on burn scar → alert users in downstream channel paths

- Earthquake M5+ in known landslide terrain → susceptibility-based alert

- Existing landslide blocking road → Exodus route modification

- Slow-moving landslide (InSAR detected) → pre-evacuation awareness alert

# BSP Encoding (Hermes)

- D20: Landslide active

- D21: Landslide — road blocked

- D22: Landslide — structures affected

- D23: Rockslide / rockfall

- D24: Mudslide / debris flow