Hazmat / Chemical Spill / Infrastructure Explosion

Jupiter Hazard Module — Beacon System Reference

Status: Planned

# Overview

Hazmat incidents range from residential propane leaks to industrial chemical releases. Beacon's Atlas has a comprehensive Explosion Hazard Zones layer covering everything that can explode or release toxic materials. Jupiter hazmat uses this pre-mapped risk layer plus real-time incident detection to deliver shelter-in-place or evacuation guidance.

# Explosion Hazard Zones Layer (Atlas)

Pre-mapped in Atlas with hazard radius and explosion type classification:

- Natural gas pipelines (PHMSA pipeline data)

- Industrial facilities with RMP (EPA Risk Management Plan) filings

- Propane tanks — residential detected via Mapillary CV, industrial from EPA RMP

- Bulk storage: ammonia, chlorine, other hazardous materials

- Nuclear facilities: 10-mile EPZ and 50-mile ingestion pathway zones

- Train routes carrying hazardous materials (PHMSA hazmat route data)

- Conservative fallback: unknown facility type defaults to largest plausible radius

# Alert Dispatch Logic

## Shelter-in-Place vs. Evacuation

- Toxic gas release upwind: shelter-in-place — seal windows and doors, turn off HVAC

- Explosive/fire hazard: evacuation with distance-based radius

- Wind direction determines shelter-in-place plume corridor

- Real-time wind from HRRR + Agora crowdsourced reports for ground truth

## Sources for Incident Detection

| Source | What It Provides |
| --- | --- |
| Local 911 dispatch feeds (where accessible) | First report of incident |
| NRC incident reporting system | Nuclear/radiological incidents |
| EPA emergency response notifications | Hazardous substance releases |
| PHMSA incident reporting | Pipeline and transportation hazmat |
| User reports (Agora) | Smell gas, visible smoke, explosion sound |
| AirNow / EPA AQS real-time | Chemical concentration spikes at monitors |

# Nuclear Hazard Zones

- 10-mile EPZ: immediate evacuation zone around each reactor

- 50-mile ingestion pathway: food/water contamination risk

- Dynamic downwind plume corridors: pre-computed for 8 wind directions × 3 stability classes

- NRC event notification system → trigger plume alert for appropriate wind direction corridor

# BSP Encoding (Hermes)

- E00–E0C: Chemical release types

- E10–E14: Fuel/oil spills

- E20–E22: Radiological/nuclear incidents

- Observation: smell gas, explosion heard, visible chemical plume, evacuation order received