World Base Map — Global Processing Pipeline

Data sources processed worldwide in a single pass, rather than region by region

HydroSHEDS Layer

Global watershed boundaries and drainage network. Combined with the Copernicus DEM to calculate how high each point sits above its nearest stream — the core input for flood depth estimation.

# Processor: hydrosheds_hand

Height Above Nearest Drainage. Meters above nearest stream at every point. A point 3m above its stream floods in a 3m+ event. Processes per-basin (water doesn’t follow tile grids). Large basins need 20–50 GB RAM.

Takes in

Global VRT:  s3://beacon-atlas/world_base_map/copernicus_dem/v{NNN}/dem/global.vrt

Basin boundaries:  atlas/world_base_map/processors/hydro_hydrosheds.py output

Spits out

HAND raster (per basin):  s3://beacon-atlas/world_base_map/hydrosheds/v{NNN}/hand/{basin_id}.tif

Visualizer tiles: None. Intermediate for chopper + landslide.

Files

Folder:  atlas/world_base_map/processors/hydrosheds_hand/

Script:  atlas/world_base_map/processors/hydrosheds_hand/hydrosheds_hand.py

Validator:  atlas/world_base_map/processors/hydrosheds_hand/validate.py

README:  atlas/world_base_map/processors/hydrosheds_hand/README.md

Updates when

Copernicus DEM reprocessed or HydroBASINS boundaries updated.

Pseudocode

1.  For each HydroBASINS level 4–6 basin:

Basins are the natural unit — water flows within basins, not tile grids

1a.  Read DEM for basin from global VRT

1b.  Breach depressions

Fill artificial pits that trap flow and break drainage

1c.  D8 flow accumulation to find streams

Traces water flow to build drainage network

1d.  Calculate elevation above nearest stream at every point

1e.  Write per-basin Float32 raster

1f.  Log success/failure, time, peak memory, size

2.  Processor exits. Orchestrator validates

3.  On pass: promote, archive

Validation checks

✓:  Basin count matches expected

✓:  All HAND ≥ 0

✓:  No basins 100% nodata where DEM valid

✓:  Stream density reasonable

✓:  Known floodplains = low HAND

Telemetry

Wall-clock time:  Total + per-basin (avg, p95, max)

Basin success rate:  Processed / attempted

Peak memory per basin:  Individually logged (flag >50 GB)

CPU hours:  Total

Storage out:  Bytes

Archive

Previous:  s3://beacon-atlas/world_base_map/archive/hydrosheds/v{NNN}/hand/

Storage class:  S3 Glacier Deep Archive

Logs

Processing log:  s3://beacon-atlas/world_base_map/update_logs/hydrosheds/v{NNN}/hand.json

Validation log:  s3://beacon-atlas/world_base_map/update_logs/hydrosheds/v{NNN}/hand_validation.json