World Base Map — Global Processing Pipeline

Data sources processed worldwide in a single pass, rather than region by region

# What This Document Covers

This document and its companion layer docs describe the data pipelines that process worldwide data sources for the world base map. These are sources that update all at once globally (e.g., Copernicus releases a new DEM for the entire world every 2–3 years) and are most effectively processed in a single pass rather than region by region.

This document covers system architecture: pipeline flow, storage layout, database schemas, execution order. The companion docs (one per data layer) contain detailed pseudocode, validation checks, telemetry, and file paths for each fetcher and processor.

# Data Sources Covered

- Copernicus GLO-30 — 30m global elevation model (~26,000 tiles). Foundation for all terrain analysis.

- HydroSHEDS / HydroBASINS — Global watershed boundaries. Used with Copernicus DEM for flood depth reference.

- Sentinel-2 — Cloudless satellite imagery mosaic. 2D base layer for users.

- OpenStreetMap — Road network. Powers Exodus evacuation routing.

- Volcanic databases — Vent locations, system classifications. Pyroclastic and lahar hazard modeling.

- Coastal & population data — Coastlines and population grids. Tsunami inundation modeling.

# Pipeline Flow (Orchestrator)

No processor or fetcher promotes data to live paths directly. An orchestrator manages the pipeline and enforces the validation gate:

1.  Orchestrator triggers the fetcher (on schedule or manually)

2.  Fetcher downloads to raw/ folder, verifies checksums, writes fetch log

3.  Orchestrator runs the fetcher’s validate.py against raw/ output

If fail: raw output quarantined, alert sent, fetch log records failure. Pipeline stops.

4.  Orchestrator triggers the processor

5.  Processor reads from raw/ (or another processor’s live output), writes to staging/

6.  Processor exits. It never touches live paths.

7.  Orchestrator runs the processor’s validate.py against staging/ output

If fail: staging quarantined, alert sent, processing log records failure. Pipeline stops.

8.  On pass: orchestrator moves previous live version to archive/, promotes staging to live

9.  Orchestrator writes the processing log and validation log

This separation means a bug in a processor can never skip validation or corrupt live data. The orchestrator is the only thing that moves data between staging, live, and archive.

# Code Layout

Every fetcher and processor follows the same folder structure:

Fetcher folder:  atlas/world_base_map/fetchers/{source_name}/

Script:  atlas/world_base_map/fetchers/{source_name}/{source_name}.py

Validator:  atlas/world_base_map/fetchers/{source_name}/validate.py

README:  atlas/world_base_map/fetchers/{source_name}/README.md

Processor folder:  atlas/world_base_map/processors/{processor_name}/

Script:  atlas/world_base_map/processors/{processor_name}/{processor_name}.py

Validator:  atlas/world_base_map/processors/{processor_name}/validate.py

README:  atlas/world_base_map/processors/{processor_name}/README.md

Each README mirrors the corresponding page in the companion document. A new team member can learn the system from either entry point.

# Data Storage Overview

Everything lives under s3://beacon-atlas/world_base_map/. This section maps the full bucket so you can understand where data lives and how it flows.

raw/ — Untouched source data

Written by fetchers. Read by processors. Never modified after download.

Copernicus DEM:  s3://beacon-atlas/world_base_map/raw/copernicus_dem/v{NNN}/

~26,000 GeoTIFF tiles, ~120 GB per release

Sentinel-2 mosaic:  s3://beacon-atlas/world_base_map/raw/sentinel2/v{NNN}/

OSM continent PBFs:  s3://beacon-atlas/world_base_map/raw/osm/v{NNN}/{continent}.pbf

Fetch manifests:  s3://beacon-atlas/world_base_map/raw/{source}/v{NNN}/manifest.json

Retention:  Kept until next version is validated and live. Then moved to archive.

staging/ — Processor output awaiting validation

Written by processors. Validated by orchestrator. Never served to users.

Pattern:  s3://beacon-atlas/world_base_map/staging/{processor_name}/v{NNN}/

Mirrors the structure of the live path.

On pass: contents moved to live. On fail: moved to quarantine.

Retention:  Temporary. Cleared after promotion or quarantine.

{source}/v{NNN}/ — Validated, live data

This is what processors read from and users see. Two most recent versions kept in standard S3 for quick rollback.

Copernicus DEM tiles:  s3://beacon-atlas/world_base_map/copernicus_dem/v{NNN}/dem/{tile_id}.tif

Copernicus VRT index:  s3://beacon-atlas/world_base_map/copernicus_dem/v{NNN}/dem/global.vrt

Slope:  s3://beacon-atlas/world_base_map/copernicus_dem/v{NNN}/slope/{tile_id}.tif

Aspect:  s3://beacon-atlas/world_base_map/copernicus_dem/v{NNN}/aspect/{tile_id}.tif

Hillshade + VRT:  s3://beacon-atlas/world_base_map/copernicus_dem/v{NNN}/hillshade/{tile_id}.tif

2D map tiles:  s3://beacon-atlas/world_base_map/copernicus_dem/v{NNN}/tiles/2d/{z}/{x}/{y}.png

3D terrain tiles:  s3://beacon-atlas/world_base_map/copernicus_dem/v{NNN}/tiles/3d/{z}/{x}/{y}.terrain

HAND:  s3://beacon-atlas/world_base_map/hydrosheds/v{NNN}/hand/{basin_id}.tif

Satellite tiles:  s3://beacon-atlas/world_base_map/sentinel2/v{NNN}/tiles/2d/{z}/{x}/{y}.jpg

archive/ — Previous versions (cold storage)

Moved here by the orchestrator when a new version is promoted. Never deleted.

Raster archives:  s3://beacon-atlas/world_base_map/archive/{source}/v{NNN}/{layer}/

DB snapshots:  s3://beacon-atlas/world_base_map/archive/{source}/v{NNN}/{table}/snapshot.sql.gz

Glacier Deep Archive:  DEM, slope, aspect, hillshade, HAND, hazard zone snapshots

Retrieval: 12–48 hrs. Cheapest. For data that rarely needs restoring.

Glacier:  Satellite tiles, road snapshots

Retrieval: 3–5 hrs. For data that may need faster rollback or comparison.

quarantine/ — Failed validation output

Kept for debugging. Contains the failed output plus the validation log showing which checks failed.

Pattern:  s3://beacon-atlas/world_base_map/quarantine/{processor_name}/v{NNN}_{timestamp}/

Retention:  Auto-deleted after 30 days unless flagged for investigation.

update_logs/ — Audit trail

Processing and validation records for every run. Always written, pass or fail.

Fetch logs:  s3://beacon-atlas/world_base_map/update_logs/{source}/v{NNN}/fetch.json

Fetch validation:  s3://beacon-atlas/world_base_map/update_logs/{source}/v{NNN}/fetch_validation.json

Processing logs:  s3://beacon-atlas/world_base_map/update_logs/{source}/v{NNN}/{processor}.json

Validation logs:  s3://beacon-atlas/world_base_map/update_logs/{source}/v{NNN}/{processor}_validation.json

Retention:  Kept indefinitely. This is the audit trail.

# PostGIS Database Tables

Live tables — Queried by downstream Beacon modules

roads — Written by osm_road_topology. Read by Exodus.

id:  Auto-increment primary key

geom:  LineString geometry of the road segment (from OSM)

highway_class:  motorway | trunk | primary | secondary | tertiary | residential | etc. (from OSM highway tag)

surface:  paved | gravel | dirt | unknown (from OSM surface tag, or inferred by processor from class + region)

source_node:  Integer. Start node ID for routing (generated by pgr_createTopology)

target_node:  Integer. End node ID for routing (generated by pgr_createTopology)

length_m:  Segment length in meters (calculated from geom)

oneway:  Boolean. Whether segment is one-way (from OSM oneway tag)

cell_id:  FK to global_cells. Which 1km grid cell this segment falls in (assigned by per-cell post-processor)

volcanic_hazard_zones — Written by volcanic_pyroclastic and volcanic_lahar. Read by Jupiter, Argus.

id:  Auto-increment primary key

geom:  Polygon geometry of the hazard zone (generated by processor from energy cone or corridor model)

hazard_type:  pyroclastic_flow | lahar (set by the writing processor)

system_id:  FK to volcanic_systems. Which volcano this zone belongs to

vent_id:  FK to volcanic_vents. Which specific vent (pyroclastic only, null for lahar)

scenario:  phreatic | small_magmatic | large (pyroclastic) or default (lahar). Set by processor.

max_reach_km:  Maximum distance the hazard extends from source (calculated by processor from model)

area_km2:  Zone area in sq km (calculated from geom)

tsunami_inundation_zones — Written by coastal_tsunami. Read by Jupiter, Argus.

id:  Auto-increment primary key

geom:  Polygon geometry of flooded area (generated by flood-fill algorithm on DEM)

wave_height_m:  2 | 5 | 10 | 20. The scenario this zone represents

area_km2:  Flooded area in sq km (calculated from geom)

max_penetration_m:  How far inland water reaches in meters (calculated from coastline to farthest flooded point)

population_exposed:  Integer. People within zone (overlay of geom with population_cells)

coastal_cell_id:  FK to global_cells. The coastline cell this inundation originates from

landslide_runout_zones — Written by copernicus_dem_landslide. Read by Jupiter, Argus.

id:  Auto-increment primary key

geom:  Polygon geometry of debris path (generated by angle-of-reach model, buffered by width)

source_geom:  Point or polygon of the initiation zone (from landslide_susceptibility)

runout_distance_m:  Total path length in meters (calculated from DEM trace)

debris_dam_risk:  Boolean. True if path crosses a stream (checked against streams table)

tsunami_risk:  Boolean. True if path reaches coast (checked against global_cells.is_coastal)

wave_risk:  Boolean. True if path reaches a lake (checked against lakes table)

area_km2:  Zone area in sq km (calculated from geom)

Reference tables — Read by processors, maintained by other pipelines

volcanic_systems — Maintained by volcanic data ingestion pipeline.

id:  Primary key

geom:  Point geometry. Volcano center location (from source database, e.g., GVP)

name:  Volcano name (from source)

has_glacier:  Boolean. Whether volcano has glacier (from source, drives lahar eligibility)

has_crater_lake:  Boolean. Whether volcano has crater lake (from source, drives lahar eligibility)

volcanic_vents — Maintained by volcanic data ingestion pipeline.

id:  Primary key

geom:  Point geometry. Vent location (from source database)

system_id:  FK to volcanic_systems. Which volcano this vent belongs to

streams — Maintained by hydro data pipeline.

id:  Primary key

geom:  LineString geometry of the stream segment (from HydroSHEDS or derived)

strahler_order:  Integer. Stream importance/size (from HydroSHEDS)

stream_cell_index — Maintained by per-cell post-processor.

stream_id:  FK to streams

cell_id:  FK to global_cells. Maps which streams pass through which grid cells

lakes — Maintained by hydro data pipeline.

id:  Primary key

geom:  Polygon geometry (from HydroLAKES or similar source)

area_km2:  Lake area in sq km (from source)

global_cells — Maintained by grid generation pipeline.

cell_id:  Primary key. Unique ID for each 1km grid cell

geom:  Polygon geometry of the cell boundary (generated by grid pipeline)

is_coastal:  Boolean. Whether cell touches ocean (derived from coastline dataset)

population_cells — Maintained by population data pipeline.

cell_id:  FK to global_cells

population:  Integer. Estimated people in this cell (from WorldPop or similar source)

landslide_susceptibility — Maintained by susceptibility data pipeline.

id:  Primary key

geom:  Polygon geometry of susceptible zone (from source dataset, e.g., NASA global landslide catalog)

level:  high | very_high (from source classification). Only these two levels are used by the landslide processor.

Staging tables — Used during processing, never queried by live systems

Pattern:  {table_name}_staging (e.g., roads_staging)

Same schema as the live table. Processor writes here. Orchestrator validates, then swaps to live.

Previous live table exported as SQL dump to archive/ before swap.

DB versioning: Tables don’t use v{NNN} paths. The processing log records input versions, and the archive snapshot provides rollback. Orchestrator handles swap atomically.

# Execution Order

Within each phase, processors run in parallel. Every validator must pass before downstream phases can start.

| Phase | Processors | Notes |
| --- | --- | --- |
| 1 | copernicus_dem_standardize | First. Everything depends on it. |
| 2 | copernicus_dem_slope, copernicus_dem_aspect, copernicus_dem_hillshade, hydrosheds_hand | All parallel. Read from full Copernicus DEM. |
| 3 | sentinel2_satellite, osm_road_topology | Independent of Phase 2. Run after fetchers. |
| 4 | volcanic_pyroclastic, volcanic_lahar, coastal_tsunami, copernicus_dem_landslide | Parallel. Need full DEM + domain tables. |

## Version Propagation

When copernicus_dem_standardize produces v003, all downstream processors write under v003:

DEM + VRT:  s3://beacon-atlas/world_base_map/copernicus_dem/v003/dem/

2D tiles:  s3://beacon-atlas/world_base_map/copernicus_dem/v003/tiles/2d/

3D tiles:  s3://beacon-atlas/world_base_map/copernicus_dem/v003/tiles/3d/

Slope / Aspect / Hillshade:  s3://beacon-atlas/world_base_map/copernicus_dem/v003/{layer}/

HAND:  s3://beacon-atlas/world_base_map/hydrosheds/v003/hand/

Independent sources maintain their own version sequences:

Sentinel-2:  s3://beacon-atlas/world_base_map/sentinel2/v012/tiles/2d/

OSM roads:  s3://beacon-atlas/world_base_map/osm/v008/

Volcanic:  s3://beacon-atlas/world_base_map/volcanic/v003/

Coastal:  s3://beacon-atlas/world_base_map/coastal/v003/

Every log records exact input versions, so any output is fully traceable.

## When to Reprocess

- New Copernicus release (~every 2–3 years): Rerun everything from Phase 1.

- New Sentinel-2 mosaic: Rerun sentinel2_satellite only.

- New OSM extract: Rerun osm_road_topology only.

- Hazard model change: Rerun the specific Phase 4 processor. DEM and intermediates reused.