Atlas — Global Base Layers

Terrain · Land Cover · Global Hydrology · Boundaries · Population · Global Hazards

Status: Fully Specced

Global layers are processed all-at-once from planet-scale data sources before any cell-level chopping. Several computations (slope, HAND, flow direction, terrain tiles) MUST run at full source resolution before cell chopping — splitting them would corrupt values at cell boundaries.

Key architectural principle: each data source produces independent estimation layers with confidence scores, never merged prematurely.

# 1. Terrain

## 1.1 DEM — Bare Earth Elevation

| Attribute | Detail |
| --- | --- |
| Layer name | dem |
| Type | Raster Float32 |
| Source | Copernicus GLO-30 (30m global); USGS 3DEP overrides in US (down to 1m) |
| Derived from | TanDEM-X radar mission (Copernicus). Buildings and vegetation edited out — bare earth only. |
| Processing | Stored as full-resolution VRT, not chopped. Must be processed globally before cell chopping. Copernicus-only: cannot get DSM from it, only bare earth. |
| Key uses | Foundation for all terrain derivatives. Flood HAND. Fire slope. Tsunami elevation lookup. Vehicle passability. Routing elevation profiles. |
| User-facing? | No — analytical foundation only |

## 1.2 Slope

| Attribute | Detail |
| --- | --- |
| Layer name | slope |
| Type | Raster Float32 (degrees 0–90) |
| Source | Derived from DEM via gdaldem slope |
| Must run at | Full source resolution before cell chopping (boundary artifacts otherwise) |
| Key uses | Fire spread rate (doubles per 10° increase). Landslide initiation (25–45° danger zone). Vehicle passability threshold. Avalanche terrain classification (>30° = avalanche terrain). |
| User-facing? | No — feeds models and passability layers |

## 1.3 Aspect

| Attribute | Detail |
| --- | --- |
| Layer name | aspect |
| Type | Raster Float32 (0–360°, -1 = flat) |
| Source | Derived from DEM via gdaldem aspect |
| Key uses | Solar exposure (south-facing slopes drier → higher fire danger). Fuel moisture modeling. Snow melt direction. Avalanche aspect-specific danger ratings. Fire spread direction. |
| User-facing? | No — analytical |

## 1.4 Hillshade

| Attribute | Detail |
| --- | --- |
| Layer name | hillshade |
| Type | Raster UInt8 |
| Source | Derived from DEM via gdaldem hillshade (azimuth 315°, altitude 45°) |
| Key uses | Visual basemap terrain layer. Default ON in app. Rendered as base tile layer in MapLibre GL JS v4.x. |
| User-facing? | 🗺️ YES — Terrain Hillshade, default ON |

## 1.5 Terrain Tiles (3D)

| Attribute | Detail |
| --- | --- |
| Layer name | terrain_tiles |
| Type | Quantized-mesh (CesiumJS format) |
| Source | Derived from DEM |
| Rendering | CesiumJS v1.119+ for 3D globe view. Self-hosted — no Cesium ion dependency (licensing restriction, prohibits offline use). |
| Key uses | 3D visualization of terrain, storm surge flooding animation, fire spread over time at neighborhood level. |
| User-facing? | 🗺️ YES — 3D view mode |

## 1.6 Surface Roughness

| Attribute | Detail |
| --- | --- |
| Layer name | surface_roughness |
| Type | Raster Float32 |
| Source | Elevation variance from DEM (rolling window) |
| Key uses | Vehicle passability (rough terrain = lower speed, higher fuel). Micro-terrain for ember deposition modeling. Hiking difficulty. |
| User-facing? | No |

## 1.7 Depression / Sink Depth

| Attribute | Detail |
| --- | --- |
| Layer name | depression_depth |
| Type | Raster Float32 (meters) |
| Source | DEM pit-filling analysis |
| Key uses | Identifies where water pools. Urban pluvial flood risk. Locating natural sinks that collect water during flash floods. |
| User-facing? | No — feeds flood model |

# 2. Land Cover

## 2.1 ESA WorldCover

| Attribute | Detail |
| --- | --- |
| Layer name | land_cover_global |
| Type | Raster UInt8 (11 classes) |
| Source | ESA WorldCover (10m, Sentinel-1 + Sentinel-2) |
| Classes | Tree cover, Shrubland, Grassland, Cropland, Built-up, Bare/sparse vegetation, Snow/Ice, Permanent water, Herbaceous wetland, Mangroves, Moss/lichen |
| Key uses | Base global land cover. Fallback for areas without LANDFIRE. Wildfire fuel type coarse classification. Flood impervious surface fraction. Global passability. |
| US replacement | LANDFIRE overrides in US at higher resolution and more fuel-relevant classification |
| User-facing? | No — feeds composite layers |

# 3. Hydrology

## 3.1 HydroSHEDS River Network

| Attribute | Detail |
| --- | --- |
| Layer name | rivers_global |
| Type | Vector lines |
| Source | HydroSHEDS (derived from SRTM DEM, globally consistent) |
| Key uses | River centerlines for HAND computation. Flood routing. Watershed delineation. Flash flood channel network for alert propagation. |
| US supplement | USGS NHD (National Hydrography Dataset) for higher resolution in US |
| User-facing? | 🗺️ YES — Rivers layer (from fused river product) |

## 3.2 HAND (Height Above Nearest Drainage)

| Attribute | Detail |
| --- | --- |
| Layer name | hand |
| Type | Raster Float32 (meters) |
| Source | Computed from DEM + river network |
| Must run at | Full source resolution before cell chopping |
| Algorithm | For each cell: vertical distance to the nearest river channel following flow paths. Not straight-line distance — follows terrain. |
| Key uses | Primary flood vulnerability index. "Gauge at 15ft → flood all cells with HAND < X meters." Quick inundation approximation without full hydraulic model. Coastal vulnerability for tsunami and storm surge. |
| US supplement | FEMA flood zones provide independent reference layer |
| User-facing? | No — feeds flood risk layer |

## 3.3 Flow Direction / Flow Accumulation

| Attribute | Detail |
| --- | --- |
| Layer name | flow_direction, flow_accumulation |
| Type | Raster Int8 / Float32 |
| Source | D8 flow routing from pit-filled DEM |
| Must run at | Full resolution before cell chopping |
| Key uses | Which way water flows from every cell. Watershed delineation. Upstream area computation. Flash flood routing — water follows these paths. |
| User-facing? | No |

## 3.4 Watershed Boundaries

| Attribute | Detail |
| --- | --- |
| Layer name | watersheds |
| Type | Vector polygons |
| Source | HydroSHEDS + USGS WBD (US) |
| Key uses | Flash flood: rainfall in watershed X → alert downstream users. Dam failure: which population is downstream? Wildfire runoff / debris flow routing post-fire. |
| User-facing? | No — operational layer for EM |

## 3.5 Lakes and Permanent Water

| Attribute | Detail |
| --- | --- |
| Layer name | water_bodies |
| Type | Vector polygons + Raster mask |
| Source | HydroSHEDS + OSM + ESA WorldCover water class + Copernicus Global Surface Water |
| Key uses | Routing (can't drive through lake). Fire modeling (water = non-fuel). Flood modeling (existing water bodies). Recreation safety (rip current, drowning hazard zones tagged separately). |
| User-facing? | 🗺️ YES — visible as blue water areas on basemap |

# 4. Global Hazard Zones

## 4.1 Fault Lines

| Attribute | Detail |
| --- | --- |
| Layer name | fault_lines |
| Type | Vector lines |
| Source | USGS Quaternary Fault Database (US); GEM Global Active Faults (global) |
| Attributes | Fault name, slip rate (mm/yr), last rupture date, fault type (normal/thrust/strike-slip), length, maximum magnitude estimate |
| Key uses | Seismic hazard display. Earthquake module trigger context. Tsunami source classification (subduction = tsunamigenic). Liquefaction susceptibility input. |
| User-facing? | 🗺️ YES — Fault Lines layer, toggleable |

## 4.2 Subduction Zone Geometry (Slab2.0)

| Attribute | Detail |
| --- | --- |
| Layer name | subduction_slabs |
| Type | Vector + depth rasters |
| Source | USGS Slab2.0 |
| Key uses | Identifies tsunamigenic earthquake sources. Used in tsunami alert trigger: M7.5+ at shallow depth on known subduction zone = immediate tsunami pipeline activation. Cascadia, Nankai, Hikurangi, Java, Cascadia pre-computed scenarios reference this geometry. |
| User-facing? | No — analytical trigger input |

## 4.3 Volcanic Hazard Zones

| Attribute | Detail |
| --- | --- |
| Layer name | volcanic_hazard_zones |
| Type | Vector polygons per hazard type |
| Source | USGS Volcano Hazards Program; Smithsonian GVP; national geological surveys |
| Sub-layers | Pyroclastic flow zones, lahar drainage paths, lava flow risk zones, ash fall isopach (historical), volcanic gas hazard areas, exclusion zones |
| Key uses | Pre-event awareness. Alert routing: lahar path → alert downstream users. Ash fall + wind direction → downwind corridor alerts. |
| User-facing? | 🗺️ YES — Volcanic Hazard Zones, toggleable |

## 4.4 Volcano Locations

| Attribute | Detail |
| --- | --- |
| Layer name | volcanoes |
| Type | Vector points |
| Source | Smithsonian GVP (Global Volcanism Program) |
| Attributes | Name, alert level (Normal/Advisory/Watch/Warning), last eruption date, volcano type, elevation, monitoring status |
| User-facing? | 🗺️ YES — Volcanoes, toggleable |

## 4.5 Landslide Susceptibility

| Attribute | Detail |
| --- | --- |
| Layer name | landslide_susceptibility |
| Type | Raster Float32 (0–1 score) |
| Source | USGS Landslide Hazards Program; NASA Global Landslide Catalog; derived from slope + geology + land cover |
| Key uses | Pre-event awareness. Post-fire: burn scar + rainfall threshold → alert users in high-susceptibility downstream channels. Earthquake trigger: M5+ shaking in susceptibility zone → alert. |
| User-facing? | 🗺️ YES — Landslide Risk, toggleable |

## 4.6 Landslide Runout Zones

| Attribute | Detail |
| --- | --- |
| Layer name | landslide_runout |
| Type | Vector polygons |
| Source | Modeled from terrain + inventory (flow path routing from susceptible slopes) |
| Key uses | Which roads and structures are in the runout path of a potential landslide. Routing: avoid roads in active runout zone. |
| User-facing? | No — feeds alert logic |

## 4.7 Tsunami Inundation Zones (Pre-Computed)

| Attribute | Detail |
| --- | --- |
| Layer name | tsunami_inundation_zones |
| Type | Vector polygons (multiple scenarios) |
| Source | NOAA pre-computed inundation scenarios for US coastal locations; HAND-derived approximation for uncatalogued areas |
| Parameterization | Multiple scenario polygons: M7.5, M8.0, M8.5, M9.0+ for local sources; PTWC-derived for far-field. Cascadia full-margin, southern segment, etc. |
| Key uses | Core tsunami alert geometry. "Your GPS location is inside the M8.5 inundation zone for a full Cascadia rupture." Alert threshold selection based on estimated magnitude. |
| User-facing? | 🗺️ YES — Tsunami Zones, toggleable |

# 5. Coastlines and Boundaries

## 5.1 Coastlines

| Attribute | Detail |
| --- | --- |
| Layer name | coastlines |
| Type | Vector lines |
| Source | OpenStreetMap (global, frequently updated); NOAA for US precision coastline |
| Key uses | Tsunami alert distance from coast. Storm surge zone definition. Coastal flood extent. Rip current beach identification. |
| User-facing? | 🗺️ YES — visible as coastline on basemap |

## 5.2 Administrative Boundaries

| Attribute | Detail |
| --- | --- |
| Layer name | admin_boundaries |
| Type | Vector polygons (multiple levels) |
| Source | OSM + Natural Earth; US: Census TIGER |
| Levels | Country, State/Province, County, City/Municipality, Census Tract (US) |
| Key uses | Alert zone definition. Emergency management jurisdiction lookup. WEA alert targeting. Legal emergency order boundaries. "You are in X County under evacuation order Y." |
| User-facing? | 🗺️ YES — admin boundaries visible at appropriate zoom levels |

# 6. Population

## 6.1 Population Density

| Attribute | Detail |
| --- | --- |
| Layer name | population_density |
| Type | Raster Float32 (people/km²) |
| Source | WorldPop (100m global); US: Census block-level disaggregated to 30m via building footprints |
| Key uses | Exposure modeling: how many people are in the path of a hazard? Alert prioritization: high-density areas get highest priority routing computation. EM resource pre-positioning. |
| User-facing? | No — analytical layer |

## 6.2 Demographic Vulnerability

| Attribute | Detail |
| --- | --- |
| Layer name | demographic_vulnerability |
| Type | Raster + Vector (census tract level) |
| Source | US Census ACS; CDC Social Vulnerability Index; EPA EJScreen |
| Attributes | Elderly population (65+) %, disability %, poverty rate, car-free households %, non-English speakers %, mobile home %, no-AC households % |
| Key uses | Identifies populations who need extra assistance during evacuations. "High density of elderly residents with no vehicle — this area needs bus/transport coordination." Feeds vulnerable population alerts. |
| User-facing? | No — EM operational layer |