Atlas — US National Source Layers

NAIP Satellite · Mapillary Street View · LANDFIRE · FEMA · Census · SSURGO

Status: Fully Specced

US national refinement layers supplement and correct global layers using US-specific high-resolution data sources. Each source generates independent estimation layers with confidence scores — never pre-merged with other sources.

# 1. NAIP Satellite Imagery (0.6m aerial)

NAIP (National Agriculture Imagery Program): 0.6m resolution 4-band (RGB + NIR) aerial imagery covering the contiguous US. Updated ~annually by state. Per-state processing with 7 extraction processors.

## 1.1 NAIP Building Footprints

| Attribute | Detail |
| --- | --- |
| Layer name | naip_buildings |
| Type | Vector GeoParquet |
| Model | U-Net semantic segmentation on NAIP |
| Attributes | Footprint polygon, roof_color (RGB), roof_material_hint (metal/shingle/flat/tile), solar_panels (bool), shadow_derived_height_estimate_m |
| Key uses | Independent building estimate for fusion. Roof color/material for fire vulnerability (dark shingles vs. metal). Solar panel detection. Where LiDAR shadow zones exist, NAIP provides building evidence. |
| User-facing? | No — feeds fused buildings layer |

## 1.2 NAIP Vegetation

| Attribute | Detail |
| --- | --- |
| Layer name | naip_vegetation |
| Type | Raster + Vector |
| Processing | Random forest classifier + U-Net on NAIP + NDVI computation |
| Outputs | Vegetation classification mask (bare/grass/shrub/tree_canopy/crop), NDVI raster, shrub polygons flagged as transitional fuel candidates, tree canopy polygons with deciduous/conifer hint |
| Key uses | Bridge fuel mapping: ornamental shrubs and hedges between structures identified as fuel connectivity. Surface fuel layer (grass = fast-spreading low-intensity, shrub = medium, canopy = crown fire potential). NDVI as live fuel moisture proxy. |
| User-facing? | No — feeds vegetation/fuel composite |

## 1.3 NAIP Surface Classification

| Attribute | Detail |
| --- | --- |
| Layer name | naip_surface |
| Type | Raster (classified) |
| Classes | Paved (dark asphalt vs. light concrete), gravel, bare soil, grass, water, pool, roof |
| Key uses | Vehicle passability: paved/gravel/firm soil vs. soft/wet. Fire modeling: impervious surfaces are fire breaks. Urban flood: impervious fraction drives runoff. |
| User-facing? | No — feeds ground surface composite |

## 1.4 NAIP Pool Detection

| Attribute | Detail |
| --- | --- |
| Layer name | naip_pools |
| Type | Vector polygons |
| Model | U-Net trained on NAIP blue tones + shape |
| Attributes | Pool polygon, estimated_volume_m³, above_ground (bool from nDSM) |
| Note | Discussed explicitly — pools visible from NAIP satellite. Not the same as water bodies (permanent) — these are residential/commercial pools. |
| Key uses | Wildfire: pools as water source for hand crew refilling. Helicopter dip tank candidates. Exposure model (above-ground pools vs. in-ground differ in flood passability). |
| User-facing? | No — operational EM layer |

## 1.5 NAIP Residential Edge

| Attribute | Detail |
| --- | --- |
| Layer name | naip_residential_edge |
| Type | Vector polygons |
| Description | Zones where residential development meets wildland vegetation — the WUI transition boundary at parcel resolution. |
| Key uses | Fine-resolution WUI boundary more accurate than LANDFIRE's WUI layer for individual parcel fire risk. Bridge fuel zone identification at residential edge. |
| User-facing? | No — feeds WUI composite layer |

## 1.6 NAIP Helicopter Landing Assessment

| Attribute | Detail |
| --- | --- |
| Layer name | naip_heli_candidates |
| Type | Vector polygons |
| Algorithm | Open area detection (no canopy, no structures from nDSM), slope < 5°, minimum size 30m×30m from NAIP open space classification |
| Key uses | Pre-screen helicopter landing zone candidates before LiDAR confirmation. Combined with LiDAR cleared areas and FAA helipad registry. |
| User-facing? | No — feeds helicopter LZ layer |

## 1.7 NAIP Coverage Confidence

| Attribute | Detail |
| --- | --- |
| Layer name | naip_coverage |
| Type | Raster |
| Description | Per-cell quality: cloud shadow, smoke obscuration, image age. Combined with LiDAR shadow coverage and Mapillary visibility to form master data confidence layer. |
| User-facing? | No — admin/QA layer |

# 2. Mapillary Street View (Ground-Level CV)

Mapillary provides crowdsourced street-level imagery with global coverage and open API. Beacon runs computer vision models on downloaded image sequences to extract ground-level features invisible from aerial sources. 7 extraction processors.

## 2.1 Street View Barriers

| Attribute | Detail |
| --- | --- |
| Layer name | streetview_barriers |
| Type | Vector GeoParquet (polylines) |
| CV models | Semantic segmentation (fence/wall/gate/guardrail/hedge class) + depth estimation for size |
| Attributes | Type (wood_fence/metal_fence/masonry_wall/hedge/gate/bollard/retaining_wall), height_m, material, is_gate (bool), locked_estimate |
| Projection | From image-space to map coordinates via Mapillary GPS + bearing + depth |
| Key uses | CRITICAL: Wooden fences are primary bridge fuel for WUI fire spread — invisible to LANDFIRE 30m raster. Confirmed fence type allows fire spread model to route ignition pathways. Gates: "Forest Service gate blocking evacuation route" — explicitly discussed (Paradise Fire, locked gates blocked evacuation). |
| User-facing? | No — feeds barrier layer and WUI fuel graph |

## 2.2 Street View Buildings and Facades

| Attribute | Detail |
| --- | --- |
| Layer name | streetview_buildings |
| Type | Vector attributes (joins to building footprints) |
| Attributes | Facade material (wood siding/stucco/brick/concrete/metal siding), window count estimate, garage presence, vent locations (attic vents = ember entry path), attached deck (combustible), attached porch |
| Key uses | Fire vulnerability: wood siding + attached deck + attic vents = high ember ignition risk. Building construction type for tornado shelter guidance. Hurricane wind vulnerability (garage doors, window count). |
| User-facing? | No — feeds structure vulnerability model |

## 2.3 Street View Vegetation

| Attribute | Detail |
| --- | --- |
| Layer name | streetview_vegetation |
| Type | Vector attributes |
| Attributes | Street tree species (conifer/deciduous/palm), ornamental shrubs near structures, hedge type, mulch presence, ground cover type |
| Key uses | Species confirmation for fuel type (conifer = higher fire danger). Ornamental vegetation as bridge fuel identification. Distance from structure confirmation. Mulch beds — slow-burning but ember-retaining surface fuel. |
| User-facing? | No — feeds vegetation/fuel composite |

## 2.4 Street View Infrastructure

| Attribute | Detail |
| --- | --- |
| Layer name | streetview_infrastructure |
| Type | Vector points |
| Detected objects | Fire hydrants (location + style), utility poles, power meter locations, propane tank candidates (cylindrical, above-ground), gas meters |
| Key uses | Hydrant locations confirmed from street view — feeds water supply layer for fire operations. Propane tank candidates cross-referenced with LiDAR cylinder detections → Explosion Hazard Zone assignment. Gas meter locations for gas shut-off guidance post-earthquake. |
| User-facing? | No — feeds infrastructure and explosion hazard layers |

## 2.5 Street View Ground Surface

| Attribute | Detail |
| --- | --- |
| Layer name | streetview_ground |
| Type | Raster attributes on road segments |
| Attributes | Pavement condition (good/cracked/potholed/unpaved), road marking visibility, shoulder type |
| Key uses | Vehicle passability: potholed/cracked road = lower speed. Unpaved confirmation. Road condition as routing edge weight modifier. |
| User-facing? | No — feeds road attribute enrichment |

## 2.6 Street View Coverage

| Attribute | Detail |
| --- | --- |
| Layer name | streetview_coverage |
| Type | Raster (visibility depth map) |
| Description | For each point in the map, how well can it be seen from available street view imagery? Accounts for buildings and vegetation blocking view. |
| Key uses | Third leg of data confidence composite (LiDAR shadow + NAIP canopy + Mapillary visibility). Where Mapillary has no coverage, weight other sources more. |
| User-facing? | No — data confidence composite |

# 3. LANDFIRE (US Fuels and Vegetation)

| Attribute | Detail |
| --- | --- |
| Layer name | landfire |
| Type | Multiple rasters (30m) |
| Source | USDA/DOI LANDFIRE program |
| Key layers | Fuel models (Scott & Burgan 40 classes), canopy cover %, canopy height m, canopy bulk density, canopy base height, existing vegetation type, existing vegetation cover, existing vegetation height, disturbance layer (burn history + harvests + insects) |
| Key uses | Primary US wildfire fuel input for ELMFIRE. 30m cells matched to ELMFIRE grid resolution. Combined with LiDAR (individual trees) and NAIP (ornamental vegetation) for sub-30m bridge fuel mapping. Disturbance layer tracks recent burns (fuel reduction), harvests, insect damage. |
| Limitations | 30m resolution misses individual trees, fences, ornamental vegetation, structures. Data lag (major fires updated 1–2 years post-event). Soil moisture NOT included — fuel moisture comes from NFDRS/weather. |
| Dynamic inputs needed | Live fuel moisture (from NFDRS stations + weather), dead fuel moisture (10-hr, 100-hr timelag), KBDI drought index, NDVI |
| User-facing? | No — fire model input |

# 4. FEMA Flood Zones (NFHL)

| Attribute | Detail |
| --- | --- |
| Layer name | fema_flood_zones |
| Type | Vector polygons |
| Source | FEMA National Flood Hazard Layer (NFHL) |
| Classes | Zone A (1% annual chance flood, no BFE), Zone AE (1% annual chance + BFE), Zone AO/AH (shallow flooding), Zone V/VE (coastal + wave action), Zone X (0.2% annual chance / minimal flood risk) |
| Attributes | Zone type, Base Flood Elevation (BFE) where available, FIRM panel date, community name |
| Key uses | Static flood risk reference layer. Insurance flood zone determination. "You are in Zone AE — 1% annual flood risk." Cross-referenced with HAND for dynamic flood extent. Limitations: static, often outdated, many communities unmapped. |
| User-facing? | 🗺️ YES — Flood Zones layer, toggleable |

# 5. Census / Demographic Data

| Attribute | Detail |
| --- | --- |
| Layer name | census_demographics |
| Type | Vector polygons (block, tract, county) |
| Source | US Census Bureau ACS (American Community Survey) + Decennial Census |
| Key attributes used | Population by block, Elderly (65+) %, Disabled %, Car-free households %, Mobile home units %, No-AC households %, Non-English speaking households %, Poverty rate, Building vintage (year built) by tract |
| Key uses | Disaggregate population to 30m cells using building footprints (building-informed dasymetric mapping). Vulnerable population alerting. PSPS medical baseline identification. Extreme heat vulnerability (no AC + elderly). Evacuation transport needs (no vehicle households). |
| User-facing? | No — analytical + EM operational layer |

# 6. SSURGO Soils

| Attribute | Detail |
| --- | --- |
| Layer name | soils_ssurgo |
| Type | Vector polygons with tabular attributes |
| Source | USDA NRCS SSURGO (Soil Survey Geographic Database) |
| Key attributes | Soil texture class (sand/silt/clay fractions), hydraulic conductivity (Ksat), available water capacity, depth to water table, erodibility (K-factor), drainage class, corrosivity |
| Key uses | Flood infiltration rate: sandy soils drain fast, clay soils pond. Landslide susceptibility: saturation capacity of cohesive soils. Wildfire: fuel moisture retention. Post-earthquake liquefaction: loose saturated sands. Vehicle passability: soft soils after rain. |
| User-facing? | No — analytical input |