Atlas — Real-Time and Dynamic Layers

Weather · Active Hazards · Road Status · Air Quality · Active Emergencies

Status: In Progress

Real-time layers are fed by Jupiter hazard models and external API feeds. They are not pre-computed in Atlas — they are overlaid on the static Atlas base at event time. Update frequencies range from seconds (ShakeAlert) to daily (drought monitor).

# 1. Weather Layers

| Layer | Source | Update Rate | Description | User-Facing? |
| --- | --- | --- | --- | --- |
| Wind speed / direction | HRRR (3km, hourly) + RTMA (real-time mesoscale analysis) | Hourly | Current and forecast wind. Primary fire spread driver. Tornado warning context. Blizzard severity. | 🗺️ YES — Wind layer, fire/weather mode |
| Temperature | HRRR + RTMA | Hourly | Current and forecast temperature. Heat index computation. Frostbite/hypothermia risk. | 🗺️ YES — Temp overlay, heat/cold events |
| Precipitation (radar) | NOAA MRMS (Multi-Radar Multi-Sensor) | Sub-hourly (~2 min) | Real-time radar-based precipitation intensity. Flash flood trigger monitoring. Landslide threshold monitoring. | 🗺️ YES — Radar/precip, default ON during storms |
| Precipitation forecast | HRRR / GFS / NBM | Hourly | Short-range QPF for flash flood risk, wildfire rain-on-snow, hurricane rainfall. | No — model input |
| Snow accumulation | SNODAS + HRRR | Daily/hourly | Real-time snow depth per road segment. Triggers passability threshold alerts. | 🗺️ YES — Snow Depth, winter events |
| Ice accretion | HRRR ice model | Hourly | Estimated ice thickness on roads and power lines. Bridge-first flagging. Power line loading risk. | 🗺️ YES — Ice Risk, winter events |
| Plow / road treatment status | State DOT APIs + crowdsourced | Variable | Time since last plow per segment. Prioritized clearing sequence. | 🗺️ YES — Plow Status, winter events |
| Lightning | Vaisala/Earth Networks lightning detection | Real-time | Strike locations and density. Backcountry safety. Fire ignition alert. | 🗺️ YES — Lightning, storm mode |
| NWS warnings/watches | NOAA CAP feed (IPAWS) | Continuous | All active NWS polygon alerts: tornado warning, flash flood, winter storm, excessive heat, etc. | 🗺️ YES — Alert polygons, always active |
| Fuel moisture | NFDRS stations + KBDI | Daily | Live and dead fuel moisture at fixed monitoring stations. Wildfire danger index. | No — fire model input |
| Smoke / AQI | AirNow + PurpleAir + HRRR-Smoke | 1-hour / real-time | Current PM2.5 at monitoring stations + sensor network + forecast plume. | 🗺️ YES — Air Quality layer |

# 2. Active Fire Layers

| Layer | Source | Update Rate | Description | User-Facing? |
| --- | --- | --- | --- | --- |
| Active fire hotspots | NASA FIRMS (VIIRS + MODIS) | ~10 min (VIIRS) | Thermal anomaly detections from satellite. Point locations with radiative power estimate. | 🗺️ YES — Fire Hotspots, default ON |
| Fire perimeter (active) | NIFC/IRWIN + CAL FIRE AGOL | Operational updates | Official fire perimeter polygons from incident teams. Accuracy varies by incident size. | 🗺️ YES — Fire Perimeter, default ON during wildfire |
| ELMFIRE spread forecast | Jupiter/wildfire model | Continuous (re-run on trigger) | Predicted fire perimeter at 1hr, 2hr, 4hr, 8hr intervals. Parameterized by current weather + fuel + topography. | 🗺️ YES — Fire Forecast, switchable confidence intervals |
| ALERTCalifornia cameras | ALERTCalifornia API | Continuous | Camera network fire detection. Human-verified sightings feed perimeter model. | No — model assimilation input |
| User fire reports | Beacon Agora | Real-time | Crowdsourced "I see fire here" reports with GPS + timestamp. Feed ELMFIRE data assimilation. | 🗺️ YES — User Reports (fire) |
| In-the-black tracking | Jupiter fire state machine | Continuous | Already-burned areas tracked as nonburnable. Safe zone state machine: unburned → burning → burned → safe. | 🗺️ YES — Fire Safe Zones |
| PSPS zones | PG&E/SCE/SDG&E PSPS APIs | Event-triggered | Active and planned public safety power shutoff zones. Combined with evacuation order alerts. | 🗺️ YES — PSPS Zones, always visible when active |

# 3. Active Flood / Water Layers

| Layer | Source | Update Rate | Description | User-Facing? |
| --- | --- | --- | --- | --- |
| Stream gauge levels | USGS Water Services API | 15 minutes | Stage height + discharge at ~10,000 US stations. Color-coded by flood stage (minor/moderate/major/record). | 🗺️ YES — Stream Gauges, water mode |
| River flood forecast | NOAA AHPS / RFC | Hourly | Stage forecasts at gauged rivers. Flood stage arrival time. | 🗺️ YES — Flood Forecast, water mode |
| Flash flood depth | Jupiter flood model (HAND-based) | Real-time | Estimated current flood depth per cell. Road crossing prioritized. Updates as gauge levels change. | 🗺️ YES — Flood Depth, active during events |
| Reservoir levels | USACE + Bureau of Reclamation APIs | Hourly | Current storage level + percent capacity. Dam failure risk trigger. | No — dam failure model input |
| Storm surge extent | Jupiter storm surge model (SLOSH-based) | Forecast-triggered | Pre-computed inundation scenarios activated by NHC track forecast. | 🗺️ YES — Storm Surge, hurricane events |
| Tsunami inundation (active) | Jupiter tsunami model | Event-triggered | Which pre-computed inundation scenario activated. ETA per coastal segment. | 🗺️ YES — Tsunami Alert, highest priority overlay |
| Tsunami wave tracking | DART buoys + PTWC | Real-time | Open ocean wave confirmation. ETA refinement from buoy detections. | No — alert refinement input |
| Electrocution risk overlay | Derived: flood extent + power lines | Event-triggered | Flood + downed lines → electrocution hazard polygon. "Do not enter this area." | 🗺️ YES — Electrocution Hazard, flood events |

# 4. Seismic / Geologic Layers

| Layer | Source | Update Rate | Description | User-Facing? |
| --- | --- | --- | --- | --- |
| Earthquake events | USGS Earthquake API | Real-time (<60 sec) | Magnitude, location, depth of all M2.0+ events. Color-coded by magnitude. | 🗺️ YES — Earthquakes, toggleable |
| ShakeAlert notification | USGS ShakeAlert | Real-time | Early warning alert with intensity estimate at user location. Seconds of lead time. | Push alert — not map layer |
| ShakeMap (post-event) | USGS ShakeMap | ~2 minutes post-event | Shaking intensity distribution (MMI + PGA + PGV). Informs building damage probability. | 🗺️ YES — Shaking Intensity, post-earthquake |
| Aftershock forecast | USGS ETAS model | Continuous post-event | Probability of aftershocks by magnitude and time window. | 🗺️ YES — Aftershock Risk, post-earthquake |
| Volcanic alert level | USGS VHP / Smithsonian | Continuous | Current alert level per volcano (Normal/Advisory/Watch/Warning). Color-coded on map. | 🗺️ YES — Volcanic Alert, toggleable |
| Lava flow extent (active) | USGS HVO + other VOs | Event-triggered | Current lava flow perimeter and advance rate. | 🗺️ YES — Lava Flow, active eruption events |
| Volcanic ash plume | VAAC + SO2 satellite | Continuous | Ash cloud position + forecast trajectory. AQI impact from volcanic particulates. | 🗺️ YES — Ash Plume, active eruption events |

# 5. Road Status and Routing Layers

| Layer | Source | Update Rate | Description | User-Facing? |
| --- | --- | --- | --- | --- |
| Road closures (official) | State DOT APIs + USFS road status + Waze | Real-time | Officially closed road segments. High priority: blocks routing engine. | 🗺️ YES — Road Closures, always visible |
| Road closures (crowdsourced) | Beacon Agora user reports | Real-time (mesh) | "Road blocked by debris / flooding / fire" reports from users. Lower priority than official until confirmed. | 🗺️ YES — Crowdsourced Road Status |
| Traffic congestion | HERE / Google traffic API + crowdsourced | Real-time | Speed and congestion. Evacuation routing: avoid congested routes. "This route is saturated." | No — routing edge weight |
| Evacuation orders | County sheriff / EM agency feeds + Zonehaven | Event-triggered | Active evacuation order and warning polygons by zone. "Zone B: mandatory evacuation order in effect." | 🗺️ YES — Evacuation Zones, highest priority |
| Contraflow status | State DOT + EM | Event-triggered | Lanes reversed for outbound evacuation. Routing engine updates edge directions. | 🗺️ YES — Contraflow, hurricane/evac events |

# 6. Infrastructure Status

| Layer | Source | Update Rate | Description | User-Facing? |
| --- | --- | --- | --- | --- |
| Power outage extent | Utility APIs (PowerOutage.us aggregator) | 15 minutes | Outage polygons with affected customer count and estimated restoration time. | 🗺️ YES — Power Outages, always visible when active |
| Cell tower status | Inferred from device behavior + carrier APIs | Real-time (inferred) | Areas where Hermes mesh network has been activated due to cell failure. Shows communication infrastructure gaps. | 🗺️ YES — Coverage Status, Hermes mode |
| Shelter capacity | FEMA NSS real-time feed + direct EM feed | Hourly during events | Current occupancy at active evacuation shelters. Prevents routing to full shelters. | 🗺️ YES — Shelter Capacity, evacuation events |