Trigger #18

Battery Death

Category: device_status

sensor_driven_triggers/device_status/battery_death/

# 1. Trigger Overview

## 1.1 What Triggers This Hazard

Battery percentage declining through documented thresholds: 50%, 20%, 10%, 5%, 2%. At 2% threshold, device enters Last Gasp state and initiates final burst transmission.

## 1.2 Associated Events Required

None required. Standalone battery monitoring. Feeds into device status tracking and emergency contact notifications.

## 1.3 Who Gets Notified

| Party | Notified | Reason |
| --- | --- | --- |
| Police | No | N/A |
| Fire | No | N/A |
| Ems | No | Only if at 2% AND active hazard event |
| Utilities | No | N/A |
| Transit | No | N/A |
| Emergency Managers | Yes | Device battery depletion status in active hazard |
| Nearby Users | No | N/A |
| Nearby Drivers | No | N/A |

## 1.4 Polling Protocol During Hazard

Battery checked every 60s under normal conditions. At each threshold crossing, transmit status update. At 2%, device enters battery conservation mode: disable WiFi, reduce GPS, lower sensor rates, then initiate Last Gasp burst at max power.

| Sensor | Frequency | Duration |
| --- | --- | --- |
| Battery monitor | 60s | Continuous |

## 1.5 End of Hazard Conditions

Device end: battery reaches 0% and device powers off. Event end: admin closes device status event.

## 1.6 Mesh Status

At 2% threshold, device broadcasts Last Gasp message via BLE with max power. Mesh relays device location and final status to hub.

## 1.7 Platform Support

| Platform | Supported | Notes |
| --- | --- | --- |
| Android | Yes | BatteryManager API for real-time level |
| iOS | Yes | UIDevice batteryLevel API |

# 2. Script Architecture

## 2.1 Sensor Watcher: watch_battery_decline.js

File: sensor_driven_triggers/device_status/battery_death/watch_battery_decline.js

Monitors battery percentage every 60s. Triggers on crossing thresholds: 50%, 20%, 10%, 5%, 2%. At each threshold, emits BATTERY_THRESHOLD_CROSSED event. At 2%, device enters Last Gasp mode.

Inputs

- battery_percent_stream

Outputs

- BATTERY_THRESHOLD_CROSSED events (5 total)

Related Code

- sensor_polling/battery_monitor.js

- sensor_driven_triggers/device_status/battery_death/battery_threshold_detector.js

- device/last_gasp_manager.js

## 2.2 Validation & Sync: validate_battery_status.js

File: sensor_driven_triggers/device_status/battery_death/validate_battery_status.js

Each threshold crossing is validated. At 2% threshold, confirms device is still functional and confirms Last Gasp sequence. No user interaction needed; automatic.

Inputs

- BATTERY_THRESHOLD_CROSSED event

Outputs

- BATTERY_STATUS_CONFIRMED event

Related Code

- device/device_health_check.js

- device/last_gasp_manager.js

## 2.3 Status Log: log_battery_status.js

File: sensor_driven_triggers/device_status/battery_death/log_battery_status.js

Creates device status record. On each threshold, updates battery_status_log. On 2% threshold, flags device as CRITICAL_BATTERY and initiates Last Gasp protocol.

Inputs

- BATTERY_STATUS_CONFIRMED event

Outputs

- battery_status_log entries

Related Code

- data/device_status_log.js

- data/battery_status_history.js

- device/last_gasp_manager.js

## 2.4 Map Icons & Notifications: notify_battery_status.js

File: sensor_driven_triggers/device_status/battery_death/notify_battery_status.js

At 2% threshold only: Emergency managers: 'Device [device_id] battery critical (2%). Last Gasp transmission sent. Location: [last_gps]. Device likely offline within minutes.' EMS/Fire/Police (if device in active hazard event): 'Device [device_id] in [hazard_name] now critical battery (2%). Position: [location]. Last update: [timestamp].'

Inputs

- BATTERY_STATUS_CONFIRMED event at 2%

- device_id

- last_known_position

Outputs

- selected_party_notifications

Related Code

- notifications/channels/notify_emergency_managers.js

- notifications/channels/notify_ems.js

- notifications/channels/notify_fire.js

- notifications/channels/notify_police.js

# 3. Alert Grouping

Function: batteryEventAggregator

File: sensor_driven_triggers/device_status/battery_death/battery_aggregator.js

- Track per-device battery state machine (prevent re-triggering on hysteresis flicker)

- Accumulate threshold events for device health monitoring (multiple low-battery events = failing battery)

- Suppress duplicate alerts within 5 min window per threshold level per device

# 4. Data Tables

## battery_threshold_events

Purpose: Threshold crossing records

Fields

- event_id

- timestamp

- device_id

- battery_pct

- threshold_crossed

- event_type

## bsp_burst_log

Purpose: Last-gasp burst transmission records

Fields

- burst_id

- timestamp

- device_id

- battery_threshold

- transmission_status

- retry_count

## battery_history

Purpose: Ongoing battery level time-series

Fields

- sample_time

- device_id

- battery_pct

- charging_status

## hazard_status_log

Purpose: Central hazard record for this trigger event

Fields

- hazard_id UUID PK

- trigger_id INT

- trigger_type TEXT

- severity ENUM(low,medium,high,critical)

- status ENUM(detected,confirmed,active,resolved,archived)

- location GEOMETRY(Point,4326)

- device_anonymous_id TEXT

- timestamp TIMESTAMPTZ

- parent_emergency_id UUID FK nullable

- sensor_snapshot_ref TEXT

- metadata_json JSONB

- created_at TIMESTAMPTZ

- resolved_at TIMESTAMPTZ nullable

## active_hazards

Purpose: Index of all currently active hazards across all trigger types

Fields

- hazard_id UUID PK FK→hazard_status_log

- trigger_type TEXT

- priority INT

- region GEOMETRY(Polygon,4326)

- device_count INT

- last_updated TIMESTAMPTZ

# 5. Hazard Model Integration

| Model | Integration | Storage |
| --- | --- | --- |
| battery_drain_rate_estimation | discharge_slope_analyzer | device_db |
| bsp_burst_transmission_reliability | network_quality_monitor | device_db |

# 6. Data Storage Practices

## Individual Event

event_db table battery_threshold_events (encrypted); bsp_burst_log (encrypted, 90-day retention)

## Emergency Linkage

Real-time battery samples in-memory only; threshold events streamed to event_db; burst stored locally until confirmed

## Retention

Threshold event records retained 30 days; battery history aggregated to daily stats after 30 days; burst logs retained 90 days

## Archival

Archive battery health metrics to device DB after 90 days; delete specific timestamps; retain only aggregated daily trends

# 7. Required Permissions

- sensor:battery_level

- sensor:charging_status

- sensor:gps (optional for last-gasp burst, requires explicit user opt-in)

- network:burst_transmission

- system:shutdown_notification

- data:event_storage

- data:local_encryption

# 8. Settings Toggle

Track battery threshold cascade (50%, 20%, 10%, 5%, 2%); emit last-gasp burst at critical 2%

# 9. Legal Concerns

- Last-gasp burst exposes location at critical moment (user in distress, hidden location, etc.)

- User may not expect location transmission at 2% battery; privacy violation

- Burst transmission consumed final power; device dies before user can manually send SOS or message

- Data retention of 2% battery events enables tracking of user movement patterns at low-power

- Burst transmission may not succeed (no network coverage); user left without notification of power loss

- Device location exposed to server at moment of power loss; enables tracking of user end-of-day position

# 10. Privacy Protections

- Battery level samples remain local; never transmitted until threshold crossed

- Threshold events anonymized: use device_id_hash only, not serial number

- Last-gasp burst location included only if user explicitly enabled location-on-shutdown in settings (default: off)

- Delete battery history older than 30 days; aggregate to daily stats only for health monitoring

- Last-gasp burst stored locally if transmission fails; requires user review before cloud upload on reboot

- Do not correlate battery death patterns with user behavior (e.g., where user ends each day)

- Redact specific location from public battery event log; store only grid cell anonymization

- User opt-in required for last-gasp burst location; default to device_id + timestamp only