# Beacon Layer Dependency Map

Every layer with its exact data sources, derived inputs, ML models, and user feedback triggers.

Generated from planning docs. 129 current layers + 50 missing layers identified.

## BEACON LAYER QUICK REFERENCE - EXACT INPUTS BY LAYER

## TERRAIN & ELEVATION

Elevation (DEM)

- PRIMARY: 3DEP LiDAR (US, 1m, 2013-2023) | FALLBACK: Copernicus DEM (Global, 30m, 2021) | FALLBACK2: ASTER GDEM (30m)

Slope Angle

- DERIVED FROM: Elevation DEM via Sobel-Scharr finite difference kernel (3×3 normalized)

Slope Aspect

- DERIVED FROM: Elevation DEM via finite difference method (compass direction 0-360°)

Terrain Curvature

- DERIVED FROM: Elevation DEM — profile curvature (perpendicular to slope) + plan curvature (across slope)

Terrain Classification

- DERIVED FROM: Slope angle thresholds — Flat (<5°) | Gentle (5-15°) | Moderate (15-30°) | Steep (30-45°) | Cliff (>45°)

Cliff Edge Detection

- PRIMARY: LiDAR slope analysis + ML (ResNet-18) | SECONDARY: Multi-directional slope analysis (slope >45°)

Traversability

- INPUTS: Elevation (DEM slope) | LiDAR DSM (obstacles) | Surface classification (vegetation/water/pavement) | 8-directional routing costs

Traversability by Vehicle Type

- INPUTS: Slope (DEM) | Surface condition (satellite + street-view) | Obstacle detection (LiDAR) | Vehicle specs (dimensions/weight)

Landslide Susceptibility

- INPUTS: Slope angle (DEM) | Terrain curvature (DEM) | Soil permeability (SSURGO) | Rainfall + drainage (HydroSHEDS)

Liquefaction Zones

- INPUTS: Vs30 shear wave velocity (USGS database) | Soil fines content (SSURGO) | Earthquake parameters (GEM faults, NSHM) | LOGISTIC REGRESSION MODEL: Zhu et al. 2017 [w1=-0.0012, w2=0.247, w3=0.0018, w4=-0.0003]

Fault Lines

- PRIMARY: GEM Global Fault Database (vector shapefile) | SUPPLEMENTAL: Fault slip rates (geodetic/paleoseismic data) | Fault dip/rake angles | Maximum magnitude estimates per fault

## HYDROLOGY & WATER

Stream Network

- DERIVED FROM: HydroSHEDS flow direction (D8 accumulation threshold >1000 cells) | SECONDARY: NHDPlus reach delineation | Strahler stream order classification

Drainage Direction

- DERIVED FROM: Elevation DEM via D8 flow direction algorithm (HydroSHEDS)

Flow Accumulation

- DERIVED FROM: Elevation DEM (D8 algorithm) | Depression-breached flow grid (to remove inland sinks)

Height Above Nearest Drainage (HAND)

- COMPUTATION: HAND = cell_elevation - nearest_stream_elevation

- INPUTS: Elevation (DEM) | Flow direction (D8, HydroSHEDS) | Stream network (from flow accumulation)

Flood Inundation Zones

- INPUTS: FEMA Flood Insurance Study (FIS) 100/500-year boundaries | FEMA NFHL BFE rasters (10m) | National Water Model (NWM) v2.1 streamflow forecasts (1km, hourly) | HEC-RAS 1D/2D hydraulic modeling | Manning's n (LiDAR canopy + substrate) | HAND raster threshold <2m | Dam inventory (USACE NID database, 91,000+ dams) | Breach scenario modeling

Watershed Boundaries

- PRIMARY: HydroSHEDS flow direction grids | SECONDARY: HydroBASINS polygons (nesting hierarchy) | USGS Watershed Boundary Dataset (WBD)

Coastlines

- DERIVED FROM: DEM/GEBCO at mean sea level (MSL) — land-water boundary delineation

Tidal Zones

- PRIMARY: NOAA tide prediction tables (mean high/low water, tidal range) | SECONDARY: GEBCO bathymetry (15 arc-sec global)

Tsunami Inundation

- INPUTS: ComMIT pre-computed tsunami scenarios (subduction zones, Mw 4.5-9.0) | GEBCO bathymetry (15 arc-sec) | Elevation DEM | Computed inundation extents per wave height (2m/5m/10m/20m scenarios)

Storm Surge Zones

- INPUTS: Historical storm tracks + intensity | Elevation DEM | Manning's n (land cover) | Computed for 100-year storm scenario

Fire Hydrants

- PRIMARY: Street-view CV detection of hydrants (ResNet-based) | SECONDARY: Utility company GIS databases | Pressure rating from utility color code

- PRIMARY: Sentinel-2 NDWI (Normalized Difference Water Index) | SECONDARY: LiDAR DSM (visible water surface) | OpenStreetMap water features | NHD/NHDPlus surface water dataset

## WATER BODIES

## SOIL & GEOLOGY

Soil Permeability

- PRIMARY: SSURGO (USDA NRCS) drainage class + hydraulic conductivity | SECONDARY: Terrain-derived (slope/aspect) | Manning's n hydrologic roughness

Permeability Classification

- DIRECT FROM: SSURGO drainage classes (well-drained | moderately well-drained | somewhat poorly drained | poorly drained)

Sinkhole Risk

- INPUTS: SSURGO depth to limestone/bedrock | Water table depth (SSURGO) | Carbonate geology (USGS Geological Survey) | Underground water flow (HydroSHEDS)

Avalanche Zones

- INPUTS: Slope angle >30° starting zones (DEM-derived) | Terrain curvature convexity/concavity adjustment | Forest density (LiDAR canopy cover, elevation >1500m) | Historical avalanche paths (state avalanche forecasts)

Volcano Monitoring Zones

- INPUTS: Smithsonian GVP volcano catalog (vent locations, eruption history, activity level) | GEM fault database (associated seismicity) | Pyroclastic flow modeling (energy cone methodology) | Lahar corridor modeling (HydroSHEDS + glacier data) | Ashfall zone computation (wind climatology)

## BUILDINGS

Footprint & Geometry

- PRIMARY: LiDAR DSM + Mask R-CNN building detection (mAP target 0.85 @ 0.5 IOU) | SECONDARY: NAIP 4-band RGB-NIR (0.6m) + U-Net segmentation | TERTIARY: OpenStreetMap building outlines | PROCESSING: Connected component polygon extraction + 0.5m simplification

Height & Floors

- INPUTS: LiDAR DSM - LiDAR DEM = height above ground | Floor count = height / 3.5m per floor average

Materials & Construction

- PRIMARY: Street-view CV (ResNet-50 material classifier) | SECONDARY: Satellite multispectral (roof material spectral signatures) | County assessor records (construction type codes)

Purpose & Use Type

- PRIMARY: OpenStreetMap building type tags | SECONDARY: Street-view CV classification | County assessor records (building class codes) | Google/OSM business hours data

Exits & Access Points

- PRIMARY: OpenStreetMap entrance/exit tags | SECONDARY: Street-view imagery (door/ramp detection) | User mobile GPS clustering

ADA Accessibility

- INPUTS: Street-view ramp detection | Building approach slope (DEM-derived) | User self-report (accessibility features)

Fire Risk Score

- FORMULA: (fuel_load × moisture_reduction_factor × vegetation_flammability) × (1 + wui_density_factor)

- INPUTS: Building materials (CV + assessor) | Spacing to neighbors (LiDAR nearest-neighbor distance) | Vegetation proximity (LANDFIRE EVT, fuel loading) | Building density (WUI density factor)

Shelter Viability

- INPUTS: Building construction type (materials, year built, code era) | Capacity (unit count, floor area) | Accessibility (exits, ADA compliance) | EMS designation (primary) | Multi-hazard vulnerability assessment (seismic, wind, flood, fire)

Seismic Fragility

- INPUTS: USGS fragility curves (by building type/material/year) | Vs30 site amplification (USGS database) | Year built / building code era (ASCE 7-22) | Soft-story detection (LiDAR structure analysis) | Seismic retrofit status (county assessor records + USGS retrofit database)

Wind Vulnerability

- INPUTS: Building height (LiDAR DSM - DEM) | Materials and roof type (street-view CV + assessor) | Exposure category (terrain roughness) | Historical wind speed data (NOAA, NWS)

Occupancy Estimates

- INPUTS: Building unit count (footprint + CV estimator) | Census household size (ACS by county) | Time-of-day multipliers (business hours, school schedules) | Seasonal adjustment (tourism, college calendars) | Transient population (hotels/hospitals/shelters — capacity counts)

Building Status

- INPUTS: EMS post-event damage reports (real-time override) | User photo confirmation (damage documentation) | Satellite imagery (large-area impact, NBR burn scars) | Building inspection records

## ROADS & TRANSPORTATION

Network Topology

- PRIMARY: OpenStreetMap road network | SECONDARY: TIGER/Line (US Census Bureau) | D-LinkNet CNN road detection (satellite validation) | LiDAR road surface detection

Road Width & Lanes

- PRIMARY: LiDAR-measured point cloud width (zonal statistics, all points) | SECONDARY: Satellite width measurement (Sentinel-2/NAIP pixel counting) | Street-view lane detection (CV for lane markings) | DOT official lane count records

Surface Type & Condition

- PRIMARY: Satellite imagery (Sentinel-2/NAIP paved vs. unpaved spectral signature) | SECONDARY: Street-view CV classification (pothole/cracking/flooding) | LiDAR reflectance patterns | User reports (seasonal passability, snow plow status)

Gates & Barriers

- PRIMARY: Street-view CV detection (gate/fence/wall material, lock type) | SECONDARY: LiDAR height measurements (barrier vertical extent) | User field surveys (gate owner contact info) | Satellite imagery (visible barriers)

Turn Radius by Vehicle

- INPUTS: Road width (LiDAR measurement) | Turn geometry (OSM angle, segment curvature) | Vehicle turning radius database (by vehicle type) | U-turn vs. K-turn feasibility indexing

Passability Model

- INPUTS: Terrain slope (DEM) | Surface condition (satellite + street-view) | Real-time weather (NOAA MRMS) | Obstacles (fallen trees, debris, flood depth) | Vehicle type constraints | Dynamic adjustments (fire perimeter blockage, flood inundation override, snow plow status)

Dead Ends & Cul-de-Sacs

- DERIVED FROM: OSM network topology (way endpoint analysis) — no outlet detection

Evacuation Routes

- COMPUTATION: Dijkstra pathfinding with multi-factor cost

- INPUTS: Road network (OSM + TIGER) | Bridge passability (height clearance, flood risk, scour stability) | Road passability (all factors above) | Vehicle-specific turn constraints | Safe zone destinations (elevation, distance, accessibility) | Congestion prediction (population density × road capacity × 0.7 cars/household) | Dynamic re-routing (fire perimeter avoidance, flood extent avoidance)

Transit Routes & Stops

- PRIMARY: GTFS (General Transit Feed Specification) from transit agencies | SECONDARY: OSM public transport tags | Real-time schedule status (transit operator API) | User reports (operational status during events)

Parking Areas

- PRIMARY: Satellite imagery (parking lot detection, space counting) | SECONDARY: LiDAR detection (flat paved surfaces >0.5 hectare) | OpenStreetMap parking tags | Street-view CV confirmation

Bridges & Overpasses

- PRIMARY: LiDAR vertical clearance measurement (bridge deck elevation) | SECONDARY: OpenStreetMap bridge geometry | DOT bridge inventory (NBIS database, weight limits, year) | NHD/NHDPlus river crossings | NFHL Base Flood Elevation (scour risk assessment) | Seismic vulnerability assessment (age, construction, site amplification)

Driveways

- INPUTS: LiDAR length/width measurements (geometry) | Slope from DEM (gradient) | Street-view visual confirmation | User-reported clearing status (post-disaster)

## VEGETATION & FUEL

Vegetation Type (EVT)

- PRIMARY: LANDFIRE Existing Vegetation Type (EVT) — 140-category raster at 30m | SECONDARY: Sentinel-2 multispectral (spectral signatures per biome) | LiDAR canopy structure validation (height, density)

Tree Species & Location

- PRIMARY: Satellite multispectral (spectral signature matching by species) | SECONDARY: LiDAR structural metrics (height, crown shape, density) | TRAINED MODEL: Random Forest on ground truth surveys | Regional forestry databases

Canopy Cover

- COMPUTATION: LiDAR returns above 2m / total returns = canopy fraction

- INPUTS: LiDAR DSM - DEM classification | Sentinel-2 NDVI | 1m/30m resolution rasterization

Canopy Height

- COMPUTATION: LiDAR Canopy Height Model (CHM) = DSM - DEM

- INPUTS: LiDAR DSM and DEM | TRAINED MODEL: Random Forest regression on LiDAR/ground survey pairs | Satellite height estimation (shadow analysis, optical stereo)

Vegetation Health (NDVI)

- COMPUTATION: NDVI = (NIR - RED) / (NIR + RED)

INPUT: Sentinel-2 multispectral (red + near-infrared bands) — 10m resolution quarterly composites

- APPLICATION: Seasonal tracking for drought stress assessment

Fire Behavior Fuel Model (FBFM)

- PRIMARY: LANDFIRE FBFM40 raster — 40 standard Rothermel fuel models at 30m | INPUTS: Vegetation type (LANDFIRE EVT) | Canopy characteristics (height, base height, bulk density)

Fuel Loading

- PRIMARY: LANDFIRE fuel loading estimates (tons/ha by class: 1hr, 10hr, 100hr, 1000hr, herbaceous, woody) | SECONDARY: LiDAR biomass estimation (allometric regression from canopy metrics) | VALIDATION: Ground surveys (destructive sampling at reference sites)

Ladder Fuel Density

- INPUTS: LiDAR vertical structure (vegetation distribution by height stratum) | Crown base height (LANDFIRE) | Understory density analysis

Fuel Moisture

- INPUTS: LANDFIRE fuel moisture classes (13 categories: 1hr, 10hr, 100hr, 1000hr, live herb, live woody, foliar) | MERRA-2 solar radiation + NOAA RH/temperature (dead fuel moisture estimation) | SMAP soil moisture + NDVI (live fuel moisture) | NFDRS Energy Release Component (ERC) percentiles (NOAA WFAS) | Interpolated weather (WindNinja or HRRR model)

Burn Scars

- COMPUTATION: Sentinel-2 Normalized Burn Ratio temporal change detection

- TRAINED MODEL: U-Net on pre/post satellite pairs

- TRAINING DATA: MTBS labeled imagery

- VALIDATION: USGS GeoMAC historical fire perimeters

Burn-Together Zones

- COMPUTATION: K-means clustering (k=5-10 per region) on building locations

- INPUTS: Building locations (LiDAR + satellite) | Structure spacing (LiDAR nearest-neighbor) | Wind direction (NOAA prevailing wind, 8 cardinal) | Fire reach probability thresholds (>0.7 = burn together group)

- RECOMPUTE: Every 15 minutes with current wind

## INFRASTRUCTURE & UTILITIES

Power Lines

- PRIMARY: LiDAR point cloud (transmission pole detection, line routing) | SECONDARY: Street-view CV (power line visibility, tower ID) | Utility company GIS maps (where available) | Satellite imagery (tower/pole localization) | TRAINED MODEL: Line detection CNN on LiDAR + labeled power lines

Power Substations

- PRIMARY: Satellite imagery (visible substation compound detection) | SECONDARY: Street-view CV (equipment classification) | Utility company GIS databases | OpenStreetMap power infrastructure tags

Power Stations

- PRIMARY: EIA (Energy Information Administration) database — plant locations, capacity, fuel type | SECONDARY: Utility GIS (generation facilities) | Satellite imagery (visual confirmation)

Utility Poles

- PRIMARY: LiDAR point cloud (vertical pole structure detection) | SECONDARY: Street-view CV (pole-mounted transformer counting) | Satellite imagery (pole localization) | Utility company GIS (pole class, age)

Water Mains

- PRIMARY: Utility company GIS databases (where available via licensing) | SECONDARY: LiDAR street subsurface clues (trench patterns) | Street-view (manhole covers, valve boxes) | Municipal GIS

Water Towers

- PRIMARY: LiDAR DSM (elevated structure detection >15m) | SECONDARY: Satellite imagery (tower silhouette) | Utility company GIS (capacity, elevation, coverage zone) | Street-view CV (visible water tank)

Gas Pipelines

- PRIMARY: PHMSA (Pipeline and Hazardous Materials Safety Administration) National Pipeline Mapping System | SECONDARY: Utility company GIS | Satellite imagery (rightof-way vegetation patterns) | Street-view (marker posts, valve stations)

Dams & Levees

- PRIMARY: NID (National Inventory of Dams) USACE database (91,000+ dams) | SECONDARY: LiDAR DSM (visible embankment, breach analysis) | Satellite imagery (reservoir extent, dam crest) | FEMA levee certification database (classification) | INPUTS: Dam height, material type, reservoir volume, breach scenarios | USACE inspection reports (safety assessment)

Hazmat Facilities

- PRIMARY: EPA Risk Management Plan (RMP) database — chemical facility locations, quantities | EPA Toxic Release Inventory (TRI) — annual chemical releases | SECONDARY: Satellite imagery (facility compound detection) | Street-view imagery (visual hazard confirmation) | Classification: fixed-quantity vs. threshold-exceeding

Nuclear Plants

- PRIMARY: NRC (Nuclear Regulatory Commission) database — reactor locations, type | SECONDARY: FEMA emergency planning zones (EPZs) — 10-mile primary, 50-mile secondary | Satellite imagery (visible reactor building)

Communication Towers

- PRIMARY: FCC database (cell tower locations, antenna counts, coverage) | SECONDARY: Satellite imagery (YOLO object detection for tower localization) | LiDAR height measurement | Network coverage mapping (signal strength interpolation)

## COMMUNICATION NETWORKS

Cell Towers

- PRIMARY: FCC database (location, antenna count, coverage) | SECONDARY: Satellite imagery (YOLO detection) | LiDAR height measurement

Satellite Terminals

- PRIMARY: Starlink/HughesNet/Viasat registries (where accessible) | SECONDARY: Street-view imagery (visible VSAT dish detection) | User reports (shelter backup communication capability)

Emergency Radio Repeaters

- PRIMARY: ARRL (American Radio Relay League) database | State emergency management frequency databases | User-deployed radio infrastructure

HAM Radio Locations

- PRIMARY: ARRL repeater database (frequencies, coverage area) | User self-report during registration

## POPULATION & DEMOGRAPHICS

Population Density

- COMPUTATION: Building unit × occupancy density aggregated to 100m grid

- INPUTS: US Census Bureau 2020 Census (block group level, primary) | Building unit count estimation (ResNet CNN on rooftop) | County household size (ACS American Community Survey)

Age Distribution

- PRIMARY: US Census Bureau demographic data (block group age cohorts: 0-17, 18-64, 65+) | SECONDARY: Nursing home locations (CMS database) + bed count | School enrollment data (NCES) — children concentration

Vulnerability Indicators

- INPUTS: CMS nursing home/hospital licensing (mobility-limited) | State health department assisted living registries | Prison/incarceration population (DOJ) | Self-reported disability (Beacon app user input) | Vehicle registration database (no-vehicle households = low-mobility proxy)

Time-of-Day Population

- COMPUTATION: Building purpose + business hours + occupancy time-series ML

- INPUTS: Building purpose (office/school/residential) | Business hours (Google/OSM for commercial) | School schedules + district calendars | Workplace employment density (Census Bureau LEHD) | Cellular anonymized location data (aggregated movement patterns) | Occupancy ML models (trained on employment/school data)

Seasonal Variation

- INPUTS: Tourism data (hotel occupancy, RV parks, campgrounds) | College academic calendars (dormitory variation) | Agricultural harvest seasons (migrant worker populations) | Ski resort seasonal staffing

Transient Population

- INPUTS: Hotel locations + room capacity (OSM, Zillow) | RV park capacity (OSM camping tags) | Campground occupancy (NPS, state parks) | Hospital bed count (CMS database) | Homeless shelter capacity (HUD database)

Language Spoken

- PRIMARY: US Census ACS data (block group primary language <5% English proficiency) | SECONDARY: User self-report (Beacon app, for personalized alerts)

## CRITICAL FACILITIES

Fire Stations

- PRIMARY: OSM amenity=fire_station tags | SECONDARY: County emergency services GIS | Satellite imagery (visible building) | ANALYSIS: Response coverage area (15-minute response standard)

EMS Stations

- PRIMARY: State EMS office databases | SECONDARY: County emergency services GIS | Ambulance dispatch records | ANALYSIS: Coverage time (8-minute response standard)

Police Stations

- PRIMARY: OSM amenity=police tags | SECONDARY: County sheriff/local PD GIS | Jurisdiction coverage mapping

Hospitals

- PRIMARY: CMS National Provider Identifier (NPI) database | SECONDARY: Disaster medical facility pre-registrations | Bed count (hospital discharge database) | Trauma center designation (state health department) | Emergency department surge capacity (historical patient volume)

Schools

- PRIMARY: NCES (National Center for Education Statistics) school database | SECONDARY: School district GIS | Enrollment data (state education agency) | Facility assessment (building condition ratings) | Special needs population (IEP students, English language learners)

Nursing Homes

- PRIMARY: CMS Medicare/Medicaid facility database | SECONDARY: State health department licensing | Bed count + patient acuity level | Evacuation capability assessment

Shelters

- PRIMARY: Red Cross shelter registry | SECONDARY: FEMA pre-designated shelter list | INPUTS: Capacity (seating, square footage, ADA accessibility) | Utility status (power, water, generators) | EMS override designations (real-time event updates)

Government Buildings

- PRIMARY: GSA (General Services Administration) building inventory (federal) | SECONDARY: County assessor tax records (county/city buildings) | Satellite imagery (administrative compound) | Function inference (building class codes)

Religious Institutions

- PRIMARY: Street-view CV (church/mosque/temple identification) | SECONDARY: OSM amenity=place_of_worship tags | Congregation size estimation (seating capacity) | Community shelter potential (open to public during disasters)

Veterinary & Animal Shelters

- PRIMARY: AVMA (American Veterinary Medical Association) member database | SECONDARY: Local animal control GIS | Shelter capacity (animal care bed count, quarantine)

Airports & Helipads

- PRIMARY: FAA airport database (ICAO, runway length, elevation) | SECONDARY: Private helipad locations (insurance databases, Google Earth) | LiDAR flat surface detection (helipad landing zone assessment) | Runway condition (satellite imagery for damage assessment)

Ports & Marinas

- PRIMARY: USACE waterway database (port locations, cargo capacity) | SECONDARY: NOAA nautical charts (channel depths, harbor infrastructure) | Satellite imagery (visible dock infrastructure) | Vessel capacity (ship registry data) | Tidal zone compatibility (water evacuation routing)

## JURISDICTIONAL BOUNDARIES

Municipal Boundaries

- PRIMARY: US Census Bureau TIGER/Line municipal boundary files | SECONDARY: State/county GIS databases | OpenStreetMap admin boundary tags

County Boundaries

- PRIMARY: US Census TIGER/Line county shapefiles | SECONDARY: USGS Geographic Names Information System (GNIS)

State Boundaries

- PRIMARY: US Census TIGER/Line state boundary files | SECONDARY: USGS state border datasets

Fire Districts

- PRIMARY: State Fire Marshal's office GIS layers | SECONDARY: County fire authority GIS | Fire station service area mapping

School Districts

- PRIMARY: NCES school district boundary files | SECONDARY: State education agency GIS | School catchment area analysis

Tribal Lands

- PRIMARY: USGS Indian Land Areas Ownership (federal/state/tribal reserve boundaries) | US Census American Indian/Alaska Native/Native Hawaiian Areas (AIANNH) | Tribal government GIS databases

Federal Land

- PRIMARY: USGS PAD-US (Protected Areas Database US) — USFS, BLM, NPS, state/local agency lands | SECONDARY: Bureau of Land Management GIS | National Forest Service administrative units | National Park Service park boundaries | Military installation boundaries (DoD)

## HAZARD RISK ZONES

Fire Hazard Severity

- INPUTS: LANDFIRE Fire Regime Groups (FRG) raster (vegetation flammability tier) | CALFIRE Fire and Resource Assessment Program (FRAP) state fire hazard severity zones | Building materials + spacing (structure flammability scoring) | Vegetation fuel loading (LANDFIRE FBFM40) | FORMULA: (fuel_load × moisture_factor × vegetation_flammability) × (1 + wui_density) | Building fire risk score (materials × spacing × vegetation_proximity) | User-reported fireproofing

Seismic Hazard Zones

- INPUTS: USGS National Seismic Hazard Map (NSHM 2023) — PGA/Sa(1s) at 500-year return | GEM Global Fault Database (fault traces, slip rates, Mmax) | Vs30 site amplification (USGS Shear Wave Velocity database) | Liquefaction susceptibility (Zhu et al. 2017) | Building fragility curves (USGS by type/year) | Soft-story detection (LiDAR structure analysis)

FEMA Flood Zones

- INPUTS: FEMA National Flood Hazard Layer (NFHL) — 100/500-year zone boundaries vectorized | Base Flood Elevation (BFE) rasters (10m) | National Water Model (NWM) v2.1 streamflow forecasts (1km, hourly) | HEC-RAS hydraulic modeling output | HAND Height Above Nearest Drainage raster | NHDPlus reach delineation + Manning's n

Nuclear Emergency Zones

- INPUTS: NRC 10-mile primary EPZ + 50-mile secondary EPZ (GeoJSON polygons) | Shelter-in-place vs. evacuation protocols (zone-dependent) | Building shielding assessment (basement depth, concrete rating) | Basement depth layer

Blast Zone Modeling

- COMPUTATION: Explosion energy release × inverse distance law

- INPUTS: EPA RMP facility quantities (chemical release amount) | PHMSA pipeline locations + scenarios (rupture blast modeling) | Building exposure assessment (proximity × fragility)

Wind History

- PRIMARY: NOAA National Wind Hazard Map — wind speed probability (500-year return) | SECONDARY: Historical hurricane tracks (NHC National Hurricane Center) | Tornado frequency data (NOAA Storm Data database, state climatologies) | Local prevailing wind direction (NOAA wind roses by region) | Seasonal wind variations (winter storm frequency, peak gust speeds)

## GLOBAL FALLBACK LAYERS

Copernicus DEM 30m

- SOURCE: ESA/Copernicus Open Data Portal

- SPECS: 30m resolution global coverage (2021) | GeoTIFF tiles (1°×1°) | EPSG:4326 | LZW compression

- USE: Fallback where 3DEP LiDAR unavailable (outside US)

Sentinel-2 Land Cover

- SOURCE: Copernicus/ESA Sentinel-2 Level 2A (10m, 20m bands)

- PROCESSING: Atmospheric correction (Sen2Cor L2A, BRDF correction) | NDVI, NBR, NDWI, NDSI spectral indices

- OUTPUT: 11-class land cover classification (ESA WorldCover-aligned) | Quarterly cloudless mosaics

OpenStreetMap Vectors

- SOURCE: OSM PBF (Protocol Buffer Format) continent extracts (Geofabrik)

- CONTENT: All highway tags (road network) | Building footprints | POIs (amenity, shop, tourism) | pgRouting topology

- UPDATE: Within 1-2 weeks of changeset

HydroSHEDS

- SOURCE: Hydrological data and maps based on Shuttle Elevation Derivatives

- SPECS: 3 arc-sec (~90m) global flow direction grid | D8 flow direction + upslope accumulation area | Raster GeoTIFF format

- PROCESSING: Depression breach fills (remove inland sinks)

ESA WorldCover 10m

- SOURCE: Copernicus/ESA WorldCover v200 (2021)

- SPECS: 10m resolution | 11 land cover classes (tree/shrub/herbaceous/cultivated/built-up/water/etc.) | Cloud-optimized GeoTIFF (COG) 3°×3° tiles

- VALIDATION: Regional NLCD/CORINE classification (US/Europe)

GEBCO Bathymetry

- SOURCE: General Bathymetric Chart of the Oceans

- SPECS: 15 arc-sec (~500m) global bathymetry grid | Elevation below sea level

- USE: Tsunami inundation modeling | GeoTIFF format, EPSG:4326

GEM Fault Database

- SOURCE: Global Earthquake Model (GEM) Global Active Faults Database

- CONTENT: Vector shapefile catalog | Fault traces (polyline) | Dip angle, rake direction, slip rate (mm/year) | Maximum magnitude per fault

- UPDATE: Annual

## DERIVED & SPECIAL CASE LAYERS

Burn-Together Zones

- COMPUTATION: K-means clustering (k=5-10 per region, adjusted for density)

- DISTANCE THRESHOLD: 30m + structure_height

- FIRE REACH PROBABILITY: If >0.7 for cluster center, entire cluster labeled "burn_together=1"

- RECOMPUTE: Every 15 minutes with current wind

Safe Zones (Wildfire)

- INPUTS: Firebreaks >50m width (USFS records, satellite imagery) | Water bodies >500m² (NHD, GEBCO) | Defensible space (Cal-Fire standards, user building status) | Storm-resistant structures (concrete/steel, assessor records + material detection) | Recent burn zones (Sentinel-2 NBR, cooling progression model) | Road access validation (network connectivity check)

- RECOMPUTE: Every 30 minutes with fire perimeter update

Safe Zones (Flood)

- COMPUTATION: HAND raster thresholded at predicted_stage + 1.5m safety buffer

- INPUTS: Bridge deck elevation validation (>peak_stage + 1m = passable) | Multi-story structures with roof access (exclude basements) | Elevated first floor validation (>BFE + 1ft freeboard)

- TEMPORAL: Validity tied to peak stage ETA | Recession rate 0.1-0.3 m/hr

Evacuation Zones (Dynamic)

- INPUTS: Fire perimeter (VIIRS + InciWeb, 15-min update) | Fire spread model projection (FARSITE/FlamMap to 4-hour horizon) | Flood inundation forecast (NWM + HEC-RAS, 15-min update) | Tsunami travel time (ComMIT scenarios, <1 hour arrival)

- STRATIFICATION: Urgent (0-2km) | Ready (2-5km) | Advisory (5-10km)

Traversability by Direction (8-Cardinal)

- COMPUTATION: Slope in 8 cardinal directions (DEM directional finite differences)

- INPUTS: Terrain obstacles in each direction (LiDAR) | Cost raster per direction (elevation + roughness)

- USE: Evacuation routing from any approach angle

Wildland-Residential Transition Line

- COMPUTATION: Building footprint boundary interpolation

- INPUTS: Building footprints (LiDAR + satellite) | Vegetation type (LANDFIRE EVT threshold)

- OUTPUT: ~100m resolution | Computed for 8 cardinal wind directions (wind-dependent crossover points)

- UPDATE: With building extraction cycle (~30 days)

Crossover Points (Wind-Directional)

- COMPUTATION: Wildland-residential transition line × fire spread trajectory intersection

- INPUTS: Transition line | Wind-dependent (8 cardinal directions)

- USE: Identifies highest-probability jump points (wildland fire → residential areas)

- APPLICATION: Arson detection (fire starting AT crossover = suspicious)

Seismic Retrofit Status

- INPUTS: County assessor foundation bolting/soft-story bracing records (primary) | USGS retrofit database (secondary) | Building code era inference (year built + jurisdiction code requirements) | Field inspection reports (where available)

Soft Story Detection

- INPUTS: LiDAR structural analysis (ground floor geometry, wall density) | Street-view CV (visible open-first-floor retail) | Assessor building class codes (soft story designation) | Foundation wall percentage (<50% = soft story)

Arson Risk Zones

- COMPUTATION: Crossover point locations (wildland-residential, 8 wind directions)

- INPUTS: Infrastructure proximity analysis (power lines, camping areas, lightning history) | Fire ignition point analysis (anomalous patterns: no natural cause + suspicious location)

- SUPPLEMENTAL: Historical arson incident database (state fire marshal records)

## Real-Time User Input & Overrides

User-Reported Changes

- INPUTS: Building damage photos (timestamp + GPS, accuracy assessment) | Road damage/blockage reports (trees, flooding, debris) | Utility outages (crowd-sourced with photo confirmation) | Fire sightings (GPS + photo + optional accelerometer confirmation) | Water depth confirmation (manual UI selection + photo) | Driveway clearing status (post-disaster) | Shelter occupancy registration (real-time occupant counts)

EMS Override Designations

- INPUTS: Official evacuation zone declarations (real-time status) | Shelter-in-place orders (county/state EM) | Road closure declarations (police/DOT) | Post-disaster damage assessment (USGS ATC-20 safety tags) | Utility restoration status (utility notification of extent/timeline)

Sensor Data (Phone-Based)

- INPUTS: Accelerometer (earthquake P/S wave detection, structural collapse) | GPS (location tracking, drift analysis under bridges) | Proximity sensor (water detection via waterproof models) | Microphone (rushing water sound, structural cracking sounds) | Battery drain anomaly (heat exposure >35°C = 20-30% increase) | Light sensor (ambient + IR intensity for flame proximity)

End of Quick Reference

# Missing Layers (Not in Sidebar)

50 layers documented in planning notes but not yet in the admin sidebar.

## High Priority

- Directional Traversability (8-direction) — DEM slope in 8 cardinal directions + LiDAR obstacles + cost raster per direction

- Wildland-Residential Transition Line — building footprints + LANDFIRE EVT threshold, computed for 8 wind directions

- Crossover Points (Wind-Directional) — transition line x fire spread trajectory intersection, used for arson detection

- Burn-Together Zones — building locations + structure spacing (LiDAR) + wind direction, k-means clustering, recomputed every 15 min

- Arson Risk Zones — crossover point locations + infrastructure proximity + ignition point analysis + historical arson data

- Safe Zones (Wildfire) — firebreaks + water bodies + defensible space + storm-resistant structures + recent burns + road access

- Safe Zones (Flood) — HAND raster thresholded at predicted stage + 1.5m buffer + bridge deck validation + elevated structures

- Dynamic Evacuation Zones — fire perimeter (VIIRS/InciWeb) + fire spread projection + flood forecast + tsunami travel time

- Wildfire Smoke Plume — real-time extent from satellite + wind dispersion model

- Oxygen Depletion Zones — fire/volcano proximity, confined space modeling

- Building Flammability Score — materials + spacing + vegetation proximity (per-structure)

- Terrain Flammability — landcover-based surface ignition potential

- Aftershock Forecast — Omori decay model from mainshock parameters

- Liquefaction Triggering Risk — real-time CSR/CRR from earthquake shaking

- Power Outage Extent — real-time utility-reported outage polygons

- Water Depth — real-time gauge + NWM predicted stage

- Debris Dam Risk — flood secondary effect from vegetation + terrain constriction

- Contamination Plume — flood-related sewage/hazmat spread modeling

- Bridge Scour Risk — flood vulnerability per bridge (age + foundation + flow velocity)

- Fire-Following-Earthquake — gas pipeline proximity + ignition cascade probability

- Soft Story Detection — LiDAR structure analysis + street-view CV + assessor codes

- Seismic Retrofit Status — county assessor records + USGS retrofit DB + code era inference

## Medium Priority

- Data Confidence/Staleness metadata — source count + resolution + age per cell

- Data Gap Zones — areas relying on <3 sources, acquisition prioritization

- Global Fallback Zone markers — cells using only global-resolution data

- Walkable/Bikeable Terrain — pedestrian-specific traversability costs

- Helicopter Landing Zones — LiDAR flat surface detection + obstacle clearance

- Height Above Ground — urban canyon modeling from LiDAR DSM

- Tree Fall Hazard — wind-triggered road blockage probability (canopy + wind)

- Pile-Up Risk Zones — traffic congestion prediction from road capacity + population

- Flame Pass-Through Rate — 8-directional fire spread velocity per cell

- In The Black Layer — post-fire burned areas safe for re-entry

- Wind Change Prediction — real-time barometric pressure gradient analysis

- Radiation Safety Rating — shelter-in-place effectiveness per building

- Survivable Zones Over Time — multi-hazard projection forward 1-4 hours

- Visibility Model — smoke/fog/dust distance estimation

- User-Deployed Sensor Locations — citizen science IoT device registry

- Drainage Classification — SSURGO drainage subset for quick flood reference

# User Input & Real-Time Override Sources

## User Reports

- Building damage photos (timestamp + GPS) — triggers building status refresh

- Road blockage reports (trees, flooding, debris) — triggers passability model refresh

- Utility outage reports (crowd-sourced with photo) — triggers infrastructure layer refresh

- Fire sightings (GPS + photo + accelerometer) — triggers hazard zone refresh

- Water depth confirmation (manual UI + photo) — triggers flood inundation refresh

- Driveway clearing status (post-disaster) — triggers driveways layer refresh

- Shelter occupancy registration — triggers shelter capacity refresh

- Accessibility self-reports (disability, age, mobility needs) — triggers vulnerability indicators refresh

- Traversability contradiction (user drove through impassable terrain) — triggers full DEM cascade refresh for tile

## EMS Overrides

- Evacuation zone declarations — real-time zone status

- Shelter-in-place orders — county/state level

- Road closure declarations — police/DOT official

- Post-disaster damage assessment (ATC-20 safety tags)

- Utility restoration status (timeline + extent)

## Phone Sensor Data

- Accelerometer — earthquake P/S wave detection, structural collapse vibration

- GPS — location + drift analysis (under bridges, tunnels)

- Proximity sensor — water detection on waterproof devices

- Microphone — rushing water sound, structural cracking

- Battery drain anomaly — heat exposure >35C causes 20-30% increase

- Light sensor — ambient + IR intensity for flame proximity detection